AEEC General Session Agenda Item 5a

Cockpit Display Subcommittee (CDS)

Draft 1 of Supplement 8 to ARINC Specification 661 Part 1

AEEC Letter: 20-041/SAI-088

Comment Received:

Submitted by: Brian Gilbert

Company:  The Boeing Company

Nature of Change: Technical

## 3.1.4

Table 3.1.4-1 – Widget Event Reference

A661_EVT_ITEM_SYNCHRONIZATION column added.

Added to MapHorz, Supplement 1 – MapVert

## 3.3

Table 3.3-1 – Type of Parameters

Added:

| bcd | Binary coded decimal digits coded on 64 bits. Each 4 bits will be interpreted as a digit to be displayed in the final string except the most significant one, which will be used for the sign (F for a negative value, and 1 for a positive value). |
| --- | --- |

#### 3.4.5.1

Table 3.4.5.1-1 – A661_MapVertItemType Standard Items Description

Added:

| Legend_Highlight | 5 | This Item is a basic Legend, but it will appear only when the associated interactive Symbol_xxx Item is highlighted. Section 3.2.9.5 describes how the LegendString is managed. |
| --- | --- | --- |
| Legend_Pop_Up | 5 | This Item is a basic Legend, but it will appear only when the user selects the associated Symbol_xxx Item. Disappearance of the Legend_Pop_Up is implementation dependent. Section 3.2.9.5 describes how the LegendString is managed. |

#### 3.4.5.2.1.3

Table 3.4.5.2.1.3-1

Added to the table name and in the table:

A661_LEGEND_HIGHLIGHT

A661_LEGEND_POP_UP

## 3.7.2

Table 3.7.2-2 – SymbolToggleButton Creation Structure

Rephrased LabelString:

From: The string terminating NULL is used as string separator.

To: String terminator NULL is used as string separator before next parameter.

## 3.8.6 and 3.8.7 and 3.8.8

Table 3.8.6-1 – DataScalingLong Parameters and Table 3.8.7-1 – DataScalingULong Parameters and Table 3.8.8-1 – DataScaling FR180 Parameters

Added to TargetParameterID:

A661_CLEAR

...

A661_BUFFER_OF_MAPITEM3D

3.8.8

Figure 3.8.8 – Scaled Value

Added to the figure: EndScaledValue = 0

3.8.9

Table 3.8.9-1 – BroadcastReceiver Parameters

Added to ParameterIdent:

A661_CLEAR

...

A661_BUFFER_OF_MAPITEM3D

3.9.8

Table 3.9.8-1 – AnimationOnParam Parameters

Added to TargetParamID

A661_CLEAR

...

A661_BUFFER_OF_MAPITEM3D

3.9.15

Added

Symbol_Target_Parkable_Interatcive

Table 3.9.15-1 – MapBoundary Parameters

Added to AlgorithmArray:

A661_ARC

A661_HORZ_LINE

A661_VERT_LINE

3.9.17

Table 3.9.17-2 – EventHandler Parameters

Added to TriggerEvent:

A661_EVT_KEY

...

A661_EVT_REQUEST_REPOSITION_ON_ITEM

4.6

Tables 4.6.14 on were all changed (moved around) and added.

I made all blue bold (Table 4.6.14-Table 4.6.89). Maybe make all of 4.6 Blue Bold so you can say the whole section was reworked for the supplement pages.

8.4

Table 8.4 renamed, added to, and rearranged Extensions to be alphabetical and the Widgets per supplement to be alphabetical. Could the whole table be blue bold, because it is all reworked? (I cannot be sure of the accuracy of the blue bold Xs from draft 1 to this version.)

Appendix E

Table E-2.2-1 – A661_ParameterStructure for the SetParameter Command

Added A661_ in the second column to SYMBOL_GENERIC