STRAWMAN 1

OF

ARINC 628, SECTION 18

CABIN AND CARGO VIDEO SURVEILLANCE

This draft dated: Dec 2018

This document is based on material submitted by various participants during the drafting process. Neither AEEC nor ARINC has made any determination whether these materials could be subject to valid claims of patent, copyright or other proprietary rights by third parties, and no representation or warranty, express or implied, is made in this regard. Any use of or reliance on this document shall constitute an acceptance thereof “as is” and be subject to this disclaimer.

This is a working paper prepared by the AEEC. It does not constitute air transport industry or ARINC approval policy, nor is it endorsed by the U.S. Federal Government, any of its agencies or others who may have participated in its preparation.

# Table of Contents

Table of Contents	2

1.0	Introduction	4

1.1	Purpose of this Document	4

1.2	Related Documents	4

1.2.1	Normative Documents	4

1.2.2	Informative Documents	5

2.0	Video Surveillance System Description	6

2.1	General description	7

2.1.1	Operation	7

2.2	Main controller	7

2.3	Network switch unit	7

2.4	Cameras (CAM)	7

2.4.1	Video/data recorder	8

Data (including video and audio) of connected cameras is recorded on a video/data recorder. Data should be encrypted (optional).	8

2.4.2	Displays	8

2.4.3	Other video sinks	8

3.0	Physical Definition	9

3.1	Main controller	9

3.1.1	Mechanical Interface	9

3.1.2	Electrical Interface	9

3.1.3	Performance	9

3.2	Network switch unit	10

3.2.1	Mechanical Interface	10

3.2.2	Electrical Interface	10

3.2.3	Performance	10

3.3	Cameras (CAM)	11

3.3.1	Mechanical Interface	11

3.3.2	Electrical Interface	11

3.3.3	Optical Interface	12

3.3.4	Performance	12

3.4	Video sinks	12

3.4.1	Video/data recorder	12

3.4.2	Displays	14

3.4.3	Further video sinks	14

4.0	Video/Audio encoding and decoding	15

4.1	Motion JPEG	15

4.2	H.264	15

4.3	H.265	15

4.4	Audio encoding	15

5.0	Video and Audio Quality	16

5.1	Video quality	16

5.1.1	Image resolution	16

5.1.2	Key image quality parameter	16

5.1.3	Frame rate	16

5.2	Audio quality	16

5.2.1	Key audio quality parameter	16

6.0	Communication	16

6.1	Video transmission	17

6.1.1	RTSP/RTP	17

6.1.2	Ancillary data	17

6.2	Control data	17

6.2.1	Configuration data	17

6.2.2	Build In Test Environment (BITE) data	17

7.0	Security Considerations	17

# Introduction

## Purpose of this Document

This Specification defines the design philosophy and the interfaces of cabin and cargo video surveillance equipment and system.

Today there is a large variety of Cabin and Cargo Surveillance Systems available with different technologies (analog, digital), different topologies, different interfaces and different performance. The customization process, adaption to cabin layouts, network wiring definition and integration of the devices are time consuming and expensive.

This specification harmonizes the design by modular architecture scalable to aircraft size and customer needs with standardized interfaces. It allows standardized provisions in the aircraft and reduces the customization effort to a minimum.

…

## Related Documents

The latest revision of the following documents is pertinent to the design of equipment intended to meet this standard. The related documents are listed in two subsections as Normative and Informative.

### Normative Documents

Documents listed in this section are directly related and referenced in the text.

ANSI/NEMA WD 6-2002: Wiring Devices – Dimensional Requirements

ARINC Specification 600: Air Transport Avionics Equipment Interfaces

ARINC Specification 615: Software Data Loader Using Ethernet Interfaces

ARINC Specification 664: Aircraft Data Networks Part 2 Ethernet Physical and Data-Link Layer Specification

ARINC Specification 664: Aircraft Data Networks Part 3 Internet-based Protocols and Services

ARINC Specification 664: Aircraft Data Network Part 5 Network Domain Characteristics and Interconnection

ARINC Specification 665: Loadable Software Standards

ARINC Specification 666: Electronic Distribution of Software

ARINC Specification 667: Guidance for the Management of Field Loadable Software

ARINC Specification 801: Fiber Optic Connectors

ARINC Specification 802: Fiber Optic Cables

ARINC Specification 803: Fiber Optic System Design Guidelines

ARINC Report 804: Fiber Optic Active Device Specification

ARINC Report 805: Fiber Optic Test Procedures

ARINC Report 806: Fiber Optic Installation and Maintenance Procedures

ARINC Specification 811: Commercial Aircraft Information Security Concepts of Operation and Process Framework

ARINC Specification 836A: Cabin Standard Enclosures

Electronic Communications Privacy Act of 1986

EN41165: Aerospace series: Connectors electrical, rectangular, with sealed and non-sealed rear, plastic housing, locking device, operating temperatures - 55 °C to 175 °C — Part 001: Technical specification

IEEE 1394: Trade Association Specifications and Technical Bulletins

MIL-DTL-38999: Connector, Electrical Circular, Miniature, High Density

Note: For all RFC references, refer to ARINC Specification 664, Part 3, Table 1.4.2.

RTCA DO-178/EUROCAE ED-12: Software Considerations in Airborne Systems and Equipment Certification

SAE ARP 4754: Certification Considerations for Highly-Integrated or Complex Aircraft Systems

TIA/EIA 626-95: Multimode Fiber Optic Link Transmission Design

IETF RFC 7826: Real time streaming protocol

IETF RFC 3550: Real time transport protocol

### Informative Documents

Documents listed in this section provide related information. They are not referenced in the text.

EN62676: Video Surveillance for use in security applications

AC 43.13: Acceptable Methods, Techniques, and Practices-Aircraft Inspection and Repair

ARINC Specification 413: Guidance for Aircraft Electrical Power Utilization & Transient Protection

ARINC Report 604: Guidance for Design and Built-in Test Equipment (BITE)

ARINC Report 606: Guidance for Electrostatic Sensitive Device Utilization and Protection

ARINC Report 607: Design Guidance for Avionics Equipment

ARINC Report 609: Design Guidance for Aircraft Electrical Power

ARINC Report 624: Design Guidance for Onboard Maintenance System (OMS)

ARINC Specification 626: Standard ATLAS for Modular Test

ARINC Specification 628: Cabin Equipment Interfaces (CEI) Part 7, Cabin Equipment Cooling General Specification

ARINC Characteristic 720: Digital Frequency/Function Selection for Airborne Electronic Equipment

ARINC Characteristic 741: Aviation Satellite Communication System

ARINC Characteristic 761: Second Generation Aviation Satellite Communication System, Aircraft Installation Provisions

ARINC Characteristic 746: Cabin Communications System

ATA Specification 100: Specification for Manufacturer’s Technical Data

EIA 364:-() Electrical Connector/Socket Test Procedures Including Environmental Classifications

Electronic Communications Privacy Act of 1986

Federal Aviation Regulation (FAR) Part 25: (Airworthiness Standards: Transport Category Airplanes)

IEC 61930: Fiber Optic Graphical Symbology

IEEE 802.1d: Internet Working – Spanning Tree Protocol

IEEE 802.1P: Traffic Class Expediting and Dynamic Multicast Filtering

RTCA DO-160/EUROCAE ED 14: Environmental Conditions and Test Procedures for Airborne Equipment

RTCA DO-199: Potential Interference to Aircraft Electronic Equipment from Devices Carried Aboard

RTCA DO-254/EUROCAE-80: Design Assurance Guidance for Airborne Electrical Hardware

RTCA DO-214: Minimum Operational Performance Standards for Aeronautical Telecommunication Network (ATN) Avionics FOR Aircraft Audio Systems and Equipment

RTCA DO-240: Audio System Characteristic and MOPS for Microphones and Audio

ONVIF Profile S for streaming video

ONVIF Profile C for basic access control

ONVIF Profile G for edge storage and retrieval

ONVIF Profile A for broader access control configuration

ONVIF Profile T (Release Candidate) for advanced video streaming

IEC 62676-4:2014 : Video surveillance systems for use in security applications - Part 4: Application guidelines

# Video Surveillance System Description

Generic Video Surveillance Systems are composed by:

- Main controller,

- Network switch unit,

- Cameras (CAM), and

- Video sinks (e.g. displays and video/data recorder)

The four elements should be considered as parts of an overall concept, which provide the possibility for easy modular adaptions. All elements are connected by network infrastructure. However, not all elements are mandatory.

Figure 1: Video System Components

## General description

Video Systems in aircraft cabin and cargo areas are intended to help the crew in order to observe hidden areas and to detect suspicious behaviour as unruly passengers, thefts or other potential threats. The video images can be selected and viewed on displays and can be recorded for later use. Further type of video sinks may be connected to the system.

The system configuration is customisable. The modularity of the system allows customisation depending on the aircraft infrastructure and aircraft layout as well as the intended surveillance objective.

### Operation

The operational constraints are the available network bandwidth and the number of cameras needed to perform video surveillance of the entire cabin area on large commercial aircrafts. This is achieved by monitoring at least 80% of the cabin area by camera views.

Smaller system configurations can be used for specific surveillance objectives (e.g. monitoring a dedicated area as galleys, doors, etc.).

## Main controller

The main controller is hosting the system configuration data and is responsible to manage the system configuration. It is establishing the communication interfaces between the elements of the system as well as between the video system and the aircraft systems. Smaller system configurations can operate without presence of a main controller if the main controller functions are implemented by other elements of the video system.

## Network switch unit

Network switch units are serving as power supply for connected cameras and connecting multiple cameras to the network infrastructure.

## Cameras (CAM)

Cameras are capturing and encoding and optionally record the monitored video and audio data. The camera can be associated with illumination. The video and audio streams are sent to the network infrastructure. The cameras can either be connected to the network infrastructure by network switch units or connected directly to the network infrastructure or connected to other element of the video system. Illumination and audio can be separated LRU’s.

## Video/data recorder

### Data (including video and audio) of connected cameras is recorded on a video/data recorder. Data should be encrypted (optional).

Displays

The displays are allowing access to the live and optional recorded video and audio to the crew in cabin and/or cockpit. The video and audio streams are decoded by the displays.

### Other video sinks

Other video sinks for specific use cases can be added to the system, e.g. remote display unit outside of the aircraft.

# Physical Definition

## Main controller

### Mechanical Interface

#### Housing

The main controller housing size shall be in accordance to ARINC600 (max 6MCU), or ARINC 836 or any type of box fully implement the interfaces as specified below as long as the housing size is not exceeding a space envelope of 200x200x325mm..

Cooling

The cooling shall be in accordance to ARINC600. In addition the DAU thermal design shall be capable to operate the main controller on ground without ventilation, e.g. when supplied by ground service power network.

### Electrical Interface

The electrical interface between the main controller and aircraft wiring is aircraft model and configuration dependent due to system scalability and level of functionality.

|  | Interface | Reference | Definition |
| --- | --- | --- | --- |
| A | Aircraft Data Network / Network Infrastructure | a) ARINC664 b) ARINC664 c) option: ARINC 429 to be added for connection to legacy aircraft | a) 100BaseTX / 1000BaseTX b) 100BaseFX / 1000BaseFX |
| B | Power supply main controller | Xxx | 115VAC / VF (360 - 800 Hz) 28VDC |
| C | Power output to Camera | a) xxx b) xxx | a) Power Over Ethernet b) 28VDC (separate wires) |
| D | Discrete input | xxx | Digital discrete input |
| E | Discrete output | xxx | Digital discrete output |

Table 1

#### Connectors

The main controller interface should include the connectors listed in x.

| Connector | Reference |
| --- | --- |
| CONNECTOR RECTANGULAR ARINC 600, size 3 | ASNE0168, CONTACT ARRANGEMENT CODE 17A |

Table 2

### Performance

#### Computation performance

The performance is configuration dependent due to system scalability and level of functionality.

The latency time between an input to the main controller initiating an output and the output from the main controller shall not exceed:

- 2 seconds for non-live video related output

- 1 second for live video related output

Network capabilities

Any Ethernet interfaces with aircraft LANs should be consistent with ARINC 628 Part 9 with regard to IP addressing, security, and protocols.

The quality of service shall be implemented to achieve minimum latency time.

#### Network securtity

IEEE 802.1X and shall support Extensible Authentication Protocol (EAP).

## Network switch unit

### Mechanical Interface

#### Housing

The dimension of the network switch unit shall comply to a maximum space envelope of 250x170x60mm. The housing shall fully implement the interfaces as specified below

The housing of the switch may be in accordance with ARINC 836A (option).

Cooling

The network switch units shall be passively cooled and respect ARINC 628 Part 7.

### Electrical Interface

The electrical interface between the network switch units and aircraft wiring is aircraft model and configuration dependent due to system scalability and level of functionality.

|  | Interface | Reference | Definition |
| --- | --- | --- | --- |
| A | Aircraft Data Network / Network Infrastructure | a) ARINC664 Part 2 b) ARINC664 Part 2 | a) 100BaseTX / 1000BaseTX b) 100BaseFX / 1000BaseFX |
| B | Data connection to camera | ARINCxxx | 100BaseTX / 1000BaseTX |
| C | Power supply network switch units | xxx | 115VAC / VF (360 - 800 Hz) 28VDC |
| D | Power output to cameras | a) xxx b) xxx | a) Power Over Ethernet b) 28VDC (separate wires) c) No power to cameras |
| E | Discrete input | xxx | Digital discrete input |

Table 3

#### Connectors

The network switch unit interface should comply to the connectors listed in Table 4Tables

|  | Connector | Reference |
| --- | --- | --- |
| A | Aircraft Data Network / Network Infrastructure | ABS1213P1102AN |
| B | Data connection to camera | ABS1340S28-06 |
| C | Power supply switch server units | EN3646A61608BN |
| D | Power output to cameras | EN3646A62041AN |

Table 4

Performance

#### Computation performance

The performance is configuration dependent due to system scalability and level of functionality.

Network capabilities

3.2.3.3 Minimum Protocols

|  | Standard | Name | Protocol |
| --- | --- | --- | --- |
| A | IEEE 802.3y IEEE 802.3ab | 100 Base-T 1000BASE-T | 100 base T 1000 Base T |
| B | IEEE 802.3af |  | Power Over Ethernet (Mode A or Mode B) |
|  | IEEE 802.3u IEEE 802.3z |  | 100 Base FX 1000 Base FX |

3.2.3.3 Network Security

Any Ethernet interfaces with aircraft LANs should be consistent with ARINC 628 Part 9 with regard to IP addressing, security, and protocols.

IEEE 802.1X and shall support Extensible Authentication Protocol (EAP).

## Cameras (CAM)

### Mechanical Interface

#### Camera Housing

The fixation of cameras installed in aircraft cabin and aircraft cargo area shall be harmonized.

- Fixation principle of cameras in aircraft cabin:

xxx

- Fixation of cameras in aircraft cargo:

xxx

- Fixation of cameras in specific locations:

Depends on the intended location and shall be harmonized to a) and b) as far as possible.

Cooling

The CAMs shall be passively cooled.

### Electrical Interface

The electrical interface between the Cameras and aircraft wiring is aircraft model and configuration dependent due to system scalability and level of functionality.

|  | Interface | Reference | Definition |
| --- | --- | --- | --- |
| A | Aircraft Data Network / Network Infrastructure or data connection to camera | a) … b) ARINC664 Part 2 c) ARINC664 Part 2 | a) 100BaseTX / 1000BaseTX, Power over ethernet b) 100BaseTX / 1000BaseTX c) 100BaseFX / 1000BaseFX |
| B | Power supply camera | a) xxx b) xxx | a) Power Over Ethernet b) 28VDC (separate wires) |

Table 5

#### Connectors

The CAM interface should comply to the connectors listed in Tables Table 6..

|  | Connector | Reference |
| --- | --- | --- |
| A | Aircraft Data Network / Network Infrastructure or data connection to camera | - 3 mm Micro MATE-N-LOK Connector AMP2-1445094-5, Pin 1 (Tx+), 2 (Tx-), 3 (Shield), 4 (Rx+) and 5 (Rx-) - ABS1145A09S03A, Pin 6 (Rx+), 7 (Rx-), 8 (Tx+) and 9 (Tx-) – Note: same connector as for B, b) - … |
| B | Power supply camera | - Identical to A, a) - ABS1145A09S03A, Pin 1 (power), pin 2 (power return) - Note: same connector as for A, b) |

Table 6

Optical Interface

Angle / Field of view is aircraft model and configuration dependent.

Performance

#### Computation performance

Camera may be able to manage

- Recording,

- illumination

…

#### Network capabilities

3.2.3.3 Minimum Protocols

|  | Standard | Name | Protocol |
| --- | --- | --- | --- |
| A | IEEE 802.3y IEEE 802.3ab | 100 Base-T 1000BASE-T | 100 base T 1000 Base T |
| B | IEEE 802.3af |  | Power Over Ethernet (Mode A or Mode B) |
|  | IEEE 802.3u IEEE 802.3z |  | 100 Base FX 1000 Base FX |

3.2.3.3 Network Security

Any Ethernet interfaces with aircraft LANs should be consistent with ARINC 628 Part 9 with regard to IP addressing, security, and protocols.

IEEE 802.1X and shall support Extensible Authentication Protocol (EAP).

## Video sinks

### Video/data recorder

Alternative recording functions may be covered by main controller, camera, display or standalone recorder unit. This chapter defines a standalone recorder unit.

#### Mechanical Interface

##### Housing

The video/data recorder housing size shall be in accordance to ARINC600, 4MCU ARINC 836A, or standalone cabin format

The weight of the video/data recorder shall not exceed 8kg.

##### Cooling

If housing is ARINC 600, the cooling shall be in accordance to ARINC600. In addition the video/data recorder thermal design shall be capable to operate the video/data recorder on ground without ventilation, e.g. when supplied by ground service power network.

#### Electrical Interface

The electrical interface (fig. xx) between the video/data recorder and aircraft wiring is aircraft model and configuration dependent due to system scalability and level of functionality.

|  | Interface | Reference | Definition |
| --- | --- | --- | --- |
| A | Aircraft Data Network / Network Infrastructure | a) ARINC664 Part 2 b) ARINC664 Part 2 | a) 100BaseTX / 1000BaseTX b) 100BaseFX / 1000BaseFX |
| B | Power supply |  | Power Over Ethernet 28VDC 115VAC / VF (360 – 800 Hz) |

Connectors

The video/data recorder interface should include the connectors listed in Tables xxx.

Table

#### Performance

##### Storage capabilities

At a minimum, the storage shall be able to cover one flight of data.

…incl. principle to get access to the storage (remov. Cartridge)

3.4.3.2 Download performance

The data extraction from the video/data recorder shall take less than 15 minutes (TBC).

This can be done either by removable recording media or Ethernet download.

The recorder device may allows playback function in the cabin (optional)

##### Computation performance

…

##### Network capabilities

…

Any Ethernet interfaces with aircraft LANs should be consistent with ARINC 628 Part 9 with regard to IP addressing, security, and protocols.

IEEE 802.1X and shall support Extensible Authentication Protocol (EAP).

### Displays

The display may be realized in the cockpit (EBAG, MFDs with Control Panel),  in the cabin or any other location on or off the aircraft.

#### Mechanical Interface

The mechanical interface of the video displays is depending on aircraft model and configuration dependent due to system scalability and level of functionality.

#### Electrical Interface

The electrical interface (fig. xx) between the video displays and aircraft wiring is aircraft model and configuration dependent due to system scalability and level of functionality.

Connects the video display to the network infrastructure.

#### Performance

##### Video display capabilities

Resolution, dpi, quality performance (colour space, brightness, contrast)

##### Computation performance

…

##### Network capabilities

…

Any Ethernet interfaces with aircraft LANs should be consistent with ARINC 628 Part 9 with regard to IP addressing, security, and protocols.

### Further video sinks

#### Mechanical Interface

The mechanical interface of the video sinks is depending on aircraft model and configuration dependent due to system scalability and level of functionality.

3.5.1.2 Cooling

Passively cooled

#### Electrical Interface

The electrical interface (fig. xx) between the video sinks and aircraft wiring is aircraft model and configuration dependent due to system scalability and level of functionality.

|  | Interface | Reference | Definition |
| --- | --- | --- | --- |
| A | Aircraft Data Network / Network Infrastructure | a) ARINC664 Part 2 b) ARINC664 Part 2 | a) 100BaseTX / 1000BaseTX b) 100BaseFX / 1000BaseFX |
| B | Power supply |  | 115VAC / VF (360 – 800 Hz) 28VDC |

Connects the video sinks to the network infrastructure.

3.5.2.1 Connectors

The Display interface should include the connectors listed in Tables xxx.

Table

#### Performance

Cockpit monitor depends on aircraft model and configuration dependent.

##### Computation performance

…Do not use main controller computation as far as possible.

##### Network capabilities

…

Any Ethernet interfaces with aircraft LANs should be consistent with ARINC 628 Part 9 with regard to IP addressing, security, and protocols.

IEEE 802.1X and shall support Extensible Authentication Protocol (EAP).

# Video/Audio encoding and decoding

## Motion JPEG

…

## H.264

…

## H.265

…

## Audio encoding

- Pulse code modulation (PCM) with sampling rate of 16kHz and quantitation of 16bps.

- MP3

# Video and Audio Quality

## Video quality

### Image resolution

- 640x480 pixel, aspect ratio 4:3 (VGA)

- 1920x1080 pixel, aspect ratio 16:9 (HD)

- 3840x2160 pixel, aspect ratio 16:9 (Ultra HD)

### Key image quality parameter

#### Signal to noise ratio

…

#### Contrast

…

#### Colour space

…

#### Dynamic range

…

### Frame rate

A frame of at least 15 frames per second (fps) at VGA and of at least 25fps at HD and Ultra HD has to be used.

## Audio quality

Refer also to 4.4.

### Key audio quality parameter

#### Signal to noise ratio

…

#### Frequency bandwidth

…

# Communication

The communication between the elements of the Video Surveillance System is based on common internet standards (see figure xx).

Figure xx

## Video transmission

### RTSP/RTP

Refer to:

- RTSP IETF RFC 7826

- RTP IETF RFC 3550

…

### Ancillary data

…(AC data, etc)

## Control data

### Configuration data

…

### Build In Test Environment (BITE) data

…

# Security Considerations

…