ARINC 424 NDB

Draft 1 of Supplement 23

Discussion/Proposal

Location Renton, Washington

July 9-11, 2019

ADS-B Airway constraints

V.0

David Atkinson, NGA

Alexis Lehr-Kwarta, NGA

SUMMARY

NGA is receiving host data referencing ADS-B Airway constraints that indicate ADS-B out required on certain airways above FL290. Suggest adding a Route Qual 2 of ‘A’ for ADS-B required above FL290

- INTRODUCTION/ BACK GROUND

- NGA is receiving source from numerous countries that are publishing ADS-B restrictions on certain airways. A year ago is was just one or two countries, but we are now seeing it in many more.

- DISCUSSION and or ACTION

- Please see the attached .ppt below for examples from host source.

- We are suggesting adding a Route Qual 2 for ADS-B required on airways.

- Changes as depicted (Track Changes is Helpful)

- Updated version:

Table 5-3 – Route Qualifier Content

| Qualifier Description | Qualifier 1 Field Content | Qualifier 2 Field Content | Qualifier 3 Field Content |
| --- | --- | --- | --- |
| GNSS Required | G |  |  |
| GNSS or DME/DME/IRU Required | F |  |  |
| GNSS, DME/DME/IRU or DME/DME Required | A |  |  |
| Equipment requirements unspecified | U |  |  |
| ADS-B Required above FL290 |  | A |  |
| FRT Required |  | R |  |
| Parallel Offset Required |  | P |  |
| TOAC Required |  | T |  |
| RNAV 10 PBN Nav Spec |  |  | W |
| RNAV 5 PBN Nav Spec |  |  | Z |
| RNAV 2 PBN Nav Spec |  |  | Y |
| RNAV 1 PBN Nav Spec |  |  | X |
| B RNAV |  |  | B |
| P RNAV |  |  | P |
| RNP 4 PBN Nav Spec |  |  | C |
| RNP 2 PBN Nav Spec |  |  | D |
| RNP 1 PBN Nav Spec |  |  | E |
| A-RNP (Advanced RNP) PBN Nav Spec |  |  | A |
| RNP 0.3 PBN Nav Spec |  |  | G |
| PBN Nav Spec unspecified |  |  | U |
| VOR/DME RNAV |  |  | V |
| Non RNAV/RNP segment in a RNAV/RNP airway |  |  | N (Note 1) |

Note 1:	The N will be coded if an airway is coded with Route Type R but includes non PBN segments. In these cases, Qualifier 1 and 2 will be blank.