1.0	Introduction	1

1.1	Purpose of this Document	1

1.2	CNS Distributed Radio Concept	1

1.3	History	1

1.4	ARINC 678 Overview	1

2.0	APPLICABLE SYSTEMS	3

2.1	Communication Systems	3

2.1.1	High Frequency (HF) Communications	3

2.1.2	Very High Frequency (VHF) Communications	3

2.1.3	Inmarsat Satellite Communications	3

2.1.4	Iridium Satellite Communications	3

2.1.5	Aeronautical Mobile Airport Communication System (AeroMACS)	3

2.1.6	L-band Digital Aeronautical Communication System (LDACS)	4

2.2	Navigation Systems	4

2.2.1	Instrument Landing System (ILS) (Localizer/Glideslope)	4

2.2.2	VHF Omni-directional Range (VOR)	4

2.2.3	Marker Beacon (MB)	4

2.2.4	Low Range Radio Altimeter (LRRA)	5

2.2.5	Global Navigation Satellite System (GNSS)	5

2.2.6	GNSS Landing System (GLS)/VHF Data Broadcast (VDB)	5

2.2.7	Distance Measuring Equipment (DME)	5

2.3	Surveillance Systems	6

2.3.1	Air Traffic Control (ATC) Transponder	6

2.3.2	Automatic Dependent Surveillance – Broadcast Out (ADS-B Out)	6

2.3.3	Traffic-Alert Collision Avoidance System (TCAS) and Airborne Collision Avoidance System (ACAS-X)	6

2.3.4	Automatic Dependent Surveillance – Broadcast In (ADS-B In)	6

2.4	Military Systems	7

3.0	SYSTEM REQUIREMENTS	8

3.1	Safety Requirements	8

3.2	Minimum Equipment List (MEL)/Dispatch Requirements	10

3.3	Regulatory Requirements	12

3.4	Radio Frequency (RF) Signal Characteristics	14

3.5	System Design Constraints	19

3.6	Digital Interface (RF-to-Processing) Constraints	22

3.7	System Processing Requirements	23

3.8	Audio Requirements	23

3.8.1	Audio Reference Documents	23

3.8.2	Bandwidth and Dynamic Range Considerations	24

3.8.3	Absolute Delay	24

3.8.4	Differential Delay	25

3.8.5	Architecture	25

3.8.6	Transport Considerations	26

3.8.7	Synchronization Considerations	28

3.8.8	Sidetone Considerations	29

3.9	Data Security	29

3.9.1	Security Assurance Level	29

3.9.2	Segregation	29

3.9.3	Radio Frequency Interference (RFI) Monitoring	29

3.9.4	Input Validation and Robustness	29

3.9.5	Tamper Resistance	29

3.9.6	Data Loading	29

3.9.7	Centralized Security Functions	29

3.9.8	Security Logging	29

4.0	OBJECTIVES AND GOALS	31

4.1	Introduction	31

4.2	Scope of This Section	31

4.3	Distributed Radio Benefits	31

4.3.1	Benefits to the Airlines	31

4.3.2	Benefits to the Airframe Manufacturer	31

4.3.3	Benefits to Equipment Manufacturers	31

4.4	Operational Objectives	31

4.5	Design Goals	32

4.5.1	Goals of Integration	32

4.5.2	Goals of Fault Tolerance	32

4.5.2.1	Performance/Integrity	32

4.5.2.2	Scheduled Maintenance	33

4.6	Equipment Packaging and Location	33

4.6.1	Weight and Volume Considerations	33

4.6.2	Location and Accessibility of Components	34

4.7	Interchangeability	34

4.8	Spares Provisioning	34

4.9	Distributed Radio Implementation	34

4.10	Distributed Radios in Flight Simulators	35

5.0	Requirements on Supporting Technologies	37

5.1	Introduction	37

5.2	ARINC 664 – Avionics Full-Duplex Switched Ethernet Network	37

5.3	ARINC 653 – Application Software Interface	37

5.4	ARINC 624 – Onboard Maintenance System	37

5.5	ARINC 661 – Cockpit Display System (CDS) Interfaces	37

5.6	ARINC 8xx – Radio Packaging Standard	38

5.7	Fiber Optic Interfaces	38

5.8	Related Documents	38

6.0	CNS DISTRIBUTED RADIO ARCHITECTURES	39

6.1	Introduction	39

6.1.1	Architecture Scope	39

6.2	Distributed Architecture [Functional Allocation Options and Proposed Nomenclature]	40

6.2.1	Central and Remote Locations	41

6.2.2	Remote Unit Options – Passive Antenna	41

6.2.3	Remote Unit Options – Active Antenna	42

6.2.4	RRU Type Definition	43

6.2.5	Antenna Type Definition	44

6.2.6	Multiple Bearer RRU Options	44

6.2.7	Consistency of Architectures Across Breadth of CNS	45

6.3	Legacy Radio Architectures (Communications)	46

6.3.1	HF Federated	47

6.3.2	VHF Federated	47

6.3.3	L-Band Satcom (Inmarsat) Federated (Compact Configurations)	47

6.3.4	L-Band Satcom (Iridium) Federated	48

6.3.5	LDACS	48

6.3.6	AeroMACS	48

6.4	Legacy Radio Architectures (Navigation)	48

6.4.1	ILS (Localizer/Glideslope) Federated	48

6.4.2	VOR Federated	48

6.4.3	Marker Beacon (MB) Federated	49

6.4.4	LRRA Federated	49

6.4.5	GNSS Federated	49

6.4.6	GLS/VDB Federated	50

6.4.7	DME Federated	50

6.5	Legacy Radio Architectures (Surveillance)	50

6.5.1	ATC Transponder and ADS-B Out	50

6.5.2	TCAS/ACAS-XA and ADS-B In	51

6.6	Hybrid Distributed Architecture	51

7.0	REMOTE RADIO UNITS (RRU)	53

7.1	Installation	53

7.2	Power	53

7.3	Environmental Requirements (In-Work)	54

7.4	Form Factors/Packaging	58

7.5	Connectors	59

7.6	Interfaces	60

7.7	Separation/Isolation	60

7.8	Interoperability	60

8.0	Antennas	61

8.1	Installation	61

8.2	Power	62

8.3	Environmental Requirements	63

8.4	Connectors	67

8.5	Interfaces	67

8.6	Separation/Isolation	67

8.7	Interoperability	67

8.8	Interchangeability	68

ATTACHMENTS

ATTACHMENT 1 Glossary Definitions	67

ATTACHMENT 2 Acronyms and Abbreviations	68

# Introduction

## Purpose of this Document

The purpose of this document is to evaluate Communication, Navigation, and Surveillance (CNS) Distributed Radio architectures and the feasibility of distributing the RF and systems processing sections to ensure the following:

Reduce cost of equipment

Reduce Size, Weight, and Power (SWaP)

Ease of aircraft integration

Growth capability built into the design

Maintain or improve system availability, reliability, and maintainability

It provides a framework to determine whether it is feasible to develop ARINC Standards that support CNS distributed radio architectures.

## CNS Distributed Radio Concept

The CNS distributed radio concept distributes the RF transmit/receive (TX/RX) sensor portion as well as much of the processing portion of the system to areas of the aircraft near the antenna. The remaining digital portion of the system can be hosted on a general purpose computing platform such as Integrated Modular Avionics (IMA).

## History

Since the mid-1990s, the air transport industry has been investigating ways to exploit new technologies for the purpose of developing smaller, lighter, and more cost effective avionics radio systems. Beginning in 1996, AEEC published ARINC Characteristic 755: Multi-Mode Receiver – Digital, which integrated the functions for Instrument Landing System, Global Navigation Satellite System (GNSS), GNSS Landing System (GLS), and Microwave Landing System (MLS).

In 2005, AEEC published ARINC Characteristic 768: Integrated Surveillance System, which integrated Air Traffic Control (ATC) Transponder, Traffic-alert Collision Avoidance System (TCAS), Weather Radar, and Terrain Awareness and Warning System (TAWS) functions.

In 2018, the AEEC recommended the development of this CNS Distributed Radio Architecture Framework document to determine the feasibility of developing ARINC standards that would separate the RF transmit/receive (TX/RX) sensor portion of specific CNS systems and the processing portion of these systems, so that reductions in cost, size, weight, and power can be realized.

## ARINC 678 Overview

This document is organized into nine sections.

Section 1.0 introduces the document and provides background information leading to its development.

Section 2.0 presents a list of Communication, Navigation, and Surveillance (CNS) systems that are addressed within this document.

Section 3.0 provides key system requirements for each of the listed CNS systems.

Section 4.0 presents the key objectives and goals.

Section 5.0 describes the expected baseline supporting technologies that are required to support the CNS distributed radio architectures.

Section 6.0 defines the possible candidates of distributed radio architectures and addresses specific Remote Radio Units (RRUs) and antenna design considerations. This section describes the expected installation environment, as well as a number of design factors including form factor, connectors, interfaces, etc.

Section 7.0 addresses specific Remote Radio Units (RRUs) design considerations.

Section 8.0 addresses specific antenna design considerations.

Section 9.0 is a summary of conclusions and recommendations for CNS distributed radio architectures.

# APPLICABLE SYSTEMS

## Communication Systems

This section provides a functional overview of the Communication Systems that are addressed in this document.

### High Frequency (HF) Communications

The High Frequency (HF) Communication (HF Comm) system permits voice, and data (if HF data radio is installed), communication over longer distances than line-of-sight communication systems. It allows communication with ground stations or with other airplanes during long over-water flights. The HF Comm system operates in the HF aeronautical frequency band of 2 MHz to 30 MHz. The number of installed HF Comm systems depends on what route is being flown and what other long range communication systems (e.g., Satcom) are onboard.

### Very High Frequency (VHF) Communications

The Very High Frequency (VHF) Communication (VHF Comm) system provides voice and data communication over line-of-sight distances. It allows communication between airplanes or between ground stations and airplanes. The VHF Communication system operates in the VHF aeronautical frequency band of 118 MHz to 137 MHz. There are typically two or three VHF Communication systems onboard the airplane.

### Inmarsat Satellite Communications

The Inmarsat satellite communications (Inmarsat SATCOM) system uses Inmarsat satellites and ground station networks to transmit and receive data and voice messages. Inmarsat SATCOM supplies higher quality data and voice message signals for passengers and crew, over longer distances than VHF/HF communication systems. The Inmarsat satellite network covers the entire earth between -82° to +82° latitude. The system operates in the aeronautical receive frequency band of 1518 MHz to 1559 MHz and transmit band of 1626.50 MHz to 1675 MHz. The number of installed Inmarsat SATCOM systems depends on what route is being flown and what other long range communication systems are onboard (e.g., HF Comm and/or Iridium SATCOM).

### Iridium Satellite Communications

The Iridium satellite communications (Iridium Satcom) system uses Iridium satellites and ground station networks to transmit and receive data and voice messages. Iridium Satcom supplies higher quality data and voice message signals for passengers and crew, over longer distances than VHF/HF communication systems. The Iridium satellite network covers the entire earth, including both poles. The system operates in the aeronautical frequency band of 1616.00 MHz to 1626.50 MHz. The number of installed Iridium Satcom systems depends on what route is being flown and what other long range communication systems are onboard (e.g., HF Comm and/or Inmarsat Satcom).

### Aeronautical Mobile Airport Communication System (AeroMACS)

Aeronautical Mobile Airport Communication System (AeroMACS) is a wireless broadband technology that supports data communications and information sharing on the airport surface for both fixed and mobile applications. Based on the WiMAX standard (IEEE 802.16e), AeroMACS operates in the protected and licensed aeronautical frequency band of 5091 MHz to 5150 MHz.

### L-band Digital Aeronautical Communication System (LDACS)

L-band Digital Aeronautical Communication System (LDACS) is an air-to-ground communications standard that is currently in development. ICAO SARPs are being developed. The LDACS deployment is still being defined.

LDACS is expected to use the 960 MHz to 1164 MHz frequency range. LDACS deployment will also need to consider the potential interference of LDACS on existing L-band systems (e.g., Distance Measuring Equipment (DME)).

## Navigation Systems

### Instrument Landing System (ILS) (Localizer/Glideslope)

The Instrument Landing System (ILS) function provides lateral (localizer) and vertical (glideslope) guidance to the runway on approach. The system uses signals from a glideslope ground station and a localizer ground station. The localizer station transmits signals to give a lateral path to keep the airplane centered on the runway. The glideslope station transmits signals to give the airplane a descent path to the touchdown point on the runway. The localizer and glideslope deviations are displayed to the flight crew on the Primary Flight Displays (PFDs), and are used by the Autopilot and TAWS systems. The ILS system provides station 1020 Hz Morse Code audio station identification signals carried by the ILS Localizer signal. The localizer station frequencies are from 108 MHz to 112 MHz. The paired glideslope station frequencies are from 328.6 MHz to 335.4 MHz. There are typically two ILS systems installed on the airplane [some aircraft models have three ILS systems installed to support Category 3B Autoland systems].

### VHF Omni-directional Range (VOR)

The VHF Omni-directional Range (VOR) system is a navigation aid that provides magnetic bearing data to a VOR ground station. The VOR system receives the ground station signal from the tuned VOR station and calculates magnetic bearing data. The data goes to various systems, including the flight deck Navigation Displays and instruments for display of the VOR bearing, and to the Flight Management System (FMS) for use in calculating the airplane’s position. The station's audio identifier is typically a three-letter string in Morse code. The VOR operates in the frequency range of 108.00 MHz to 117.95 MHz. Single or dual VOR systems may be installed on the airplane.

### Marker Beacon (MB)

The Marker Beacon (MB) system provides visual and aural indications when the airplane flies over ground-based marker beacon transmitters. It is used to determine the airplane’s position along an established route to a destination (e.g., a runway). The MB receives only 75 MHz signals with modulations corresponding to the following MB audio outputs:

Outer Marker (OM) is 400 Hz, continuous dashes (- - - -)

Middle Marker (MM) is 1300 Hz, alternate dots and dashes (-.-.-.-.-)

Inner Marker (IM) is 3000 Hz, continuous dots (......)

Backcourse marker is 3000 Hz, continuous paired dots (.. .. .. ..)

Airways marker is 3000 Hz with the Morse code identifier for that station.

There is typically just one Marker Beacon system installed on the airplane.

### Low Range Radio Altimeter (LRRA)

The Low Range Radio Altimeter (LRRA) system measures the distance from the airplane to the ground and outputs radio altitude data. The LRRA system has a range of -20 to 5000 feet. The radio altitude is typically displayed on the pilot’s Primary Flight Display (PFD). The flight crew and other airplane systems (e.g., Autopilot, TCAS, Predictive Windshear, etc.) use radio altitude during approach and landing. It is also used as an input source by the Terrain Awareness and Warning System (TAWS) to prevent collisions into terrain. There are typically two LRRA systems installed on the airplane [some aircraft types have three LRRA systems installed to support Category 3B Autoland systems].

### Global Navigation Satellite System (GNSS)

The Global Navigation Satellite System (GNSS) receives navigation satellites’ signals to calculate accurate airplane position, altitude, velocity, and time data which can be used by a number of airplane systems (e.g., FMS, TAWS, ADS-B, etc.). To-date, the most often used navigation satellite system is the United States’ Global Positioning System (GPS) signal on L1 (1575.42 MHz +/-10 MHz) for civilian use. GPS satellites already transmit, or plan to transmit, signals for civilian use on the L5 (1176.45 MHz) frequency. There are other global navigation satellite constellations that are already in-work or planned, including: Europe’s Galileo and China’s Beidou, both of which have signals that also operate on 1575.42 MHz and 1176.45 MHz. GNSS services may also be provided by Russia’s GLONASS constellation that transmit on the L1 (1602 MHz) and L2 (1246 MHz) frequencies. In addition, Satellite Based Augmentation Systems (SBAS) operate on the L1 frequency signal and will provide additional data on the L5 frequency signal to improve GNSS performance and ensure data integrity.

Typically, there are two GNSS systems installed on the airplane. Three GNSS systems may be installed to provide GNSS Landing System (GLS) outputs to support some airplane models’ Autoland systems.

### GNSS Landing System (GLS)/VHF Data Broadcast (VDB)

The GNSS Landing System (GLS) function provides lateral and vertical guidance to the runway on approach. GLS receives both GNSS differential correction data along with approach path data from a Ground Based Augmentation System (GBAS) ground station via the VHF Data Broadcast (VDB) receiver. The VDB receiver operates in the frequency range of 108.00 MHz to 117.95 MHz. The airplane’s VDB antenna can be either the same antenna that is used for the VOR function or the same antenna that is used for the ILS localizer function, or both, since they operate in the same frequency range and have the same horizontal polarization. Typically, there are two GLS systems installed on the airplane. Three GLS systems may be installed to provide triplex GLS deviation outputs to support some airplane models’ Autoland systems.

### Distance Measuring Equipment (DME)

The Distance Measuring Equipment (DME) function provides sIant-range (line-of-sight) distance from an airplane to a selected DME ground station [DME ground stations are typically collocated with VHF Omni-directional Range (VOR) or Instrument Landing System (ILS) ground stations]. The DME distance can be presented to the flight crew on a flight deck display or instrument. DME frequency scanning mode can be used to provide multiple (up to 5 stations) distances and frequency data to the Flight Management System (FMS) for navigation purposes. The DME system also provides station audio information (1350 Hz Morse code 
3-letter identity) to the flight crew. DME operates in the L-Band frequency range: 1025 MHz to 1150 MHz (transmit) and 962 MHz to 1215 MHz (receive). The DME ground station replies on a frequency that is either 63 MHz lower or 63 MHz higher than the interrogated frequency. There are typically two DME systems installed on the airplane.

## Surveillance Systems

### Air Traffic Control (ATC) Transponder

The Air Traffic Control (ATC) Transponder function replies to 1030 MHz interrogations from ground-based Secondary Surveillance Radars (SSRs) and from airborne TCAS systems. Mode A replies provide the aircraft’s 4096 (aka squawk) code entered by the flight crew. Mode C replies provide the airplane’s uncorrected barometric altitude. Mode S replies provide various aircraft data, including flight identification, heading, track, ground speed, Traffic Collision Avoidance System TCAS status, and other data. There are two ATC Transponder systems installed, but only one is active at any given time (the other is a hot spare).

### Automatic Dependent Surveillance – Broadcast Out (ADS-B Out)

The Automatic Dependent Surveillance – Broadcast Out (ADS-B Out) function automatically transmits position, velocity, altitude, aircraft identification, and other pertinent aircraft systems data which can be received and used by ground-based ATC receivers (for separation services) and by other aircraft that have ADS-B In receivers (for use by ADS-B In applications). ADS-B Out transmits data on 
1090 MHz signals both on the ground and in the air. The ADS-B Out function is typically collocated with the ATC Transponder function which transmits interrogation replies on 1090 MHz. There are two ADS-B Out systems installed, but only one is active at any given time (the other is a hot spare).

### Traffic-Alert Collision Avoidance System (TCAS) and Airborne Collision Avoidance System (ACAS-X)

The Traffic-Alert and Collision Avoidance System (TCAS) helps the flight crew maintain safe separation from other ATC transponder equipped airplanes. TCAS is an airborne system that operates independently of the ground-based ATC system. TCAS sends 1030 MHz interrogation signals to nearby ATC transponder equipped airplanes which respond to these interrogations via 1090 MHz replies. TCAS provides a traffic display to the flight crew as well as Traffic Advisories (TAs) to alert the crew of closing aircraft. In addition, TCAS provides Resolution Advisories (RAs) that provide vertical guidance (e.g., climb or descend) audio and visual commands to the crew when a potential collision is determined. There are typically one or two TCAS systems onboard, but only one TCAS is operational at any given time (the other system is a hot spare).

### Automatic Dependent Surveillance – Broadcast In (ADS-B In)

The Automatic Dependent Surveillance – Broadcast In (ADS-B In) function receives 1090 MHz ADS-B signals from other aircraft. These signals can be used in various ADS-B In applications including, but not limited to:

Airborne Situational Awareness/Cockpit Display of Traffic Information (AIRB/CDTI)

In Trail Procedure (ITP)

Surface Situational Awareness (SURF)

CDTI Assisted Visual Separation (CAVS)

Flight-deck Interval Management (FIM)

The ADS-B In receive function is typically collocated with the TCAS function, since both require the reception of 1090 MHz signals. There are typically one or two 
ADS-B In systems onboard, but only one ADS-B In receiver is operational at any given time (the other is a hot spare).

## Military Systems

Communication, Navigation, and Surveillance (CNS) systems that specifically support military-specific functions (e.g., Tactical Air Navigation (TACAN) and Interrogate Friend or Foe (IFF)) are not addressed within this document. However, note that some military aircraft are derived from commercial aircraft platforms. Therefore, when designing the system integration of the distributed radio architecture, consideration should be given where the commercial CNS functions may integrate with the military-specific CNS functions. For example, when designing the RF suppression network between commercial L-band systems (e.g., DME, ATC Transponder, and TCAS/ACAS), consideration should also be given for RF suppression interfaces to TACAN and IFF systems.

# SYSTEM REQUIREMENTS

## Safety Requirements

The safety requirements for the applicable Communication, Navigation, and Surveillance (CNS) systems identified in this report are documented as failure classifications based on a Functional Hazard Assessment (FHA) of each system. The failure classifications are further divided between loss of function hazards and misleading/erroneous data hazards. The different classifications of failure conditions are documented in FAA/JAA Advisory Circular AC/AMJ 25.1309 (Arsenal version, dated 6/10/2002) as follows (in increasing order of severity):

No Safety Effect: Failure Conditions that would have no effect on safety; for example, Failure Conditions that would not affect the operational capability of the airplane or increase crew workload.

Minor: Failure Conditions which would not significantly reduce airplane safety, and which involve crew actions that are well within their capabilities. Minor Failure Conditions may include, for example, a slight reduction in safety margins or functional capabilities, a slight increase in crew workload, such as routine flight plan changes, or some physical discomfort to passengers or cabin crew.

Major: Failure Conditions which would reduce the capability of the airplane or the ability of the crew to cope with adverse operating conditions to the extent that there would be, for example, a significant reduction in safety margins or functional capabilities, a significant increase in crew workload or in conditions impairing crew efficiency, or discomfort to the flight crew, or physical distress to passengers or cabin crew, possibly including injuries

Hazardous: Failure Conditions which would reduce the capability of the airplane or the ability of the crew to cope with adverse operating conditions to the extent that there would be:

  - A large reduction in safety margins or functional capabilities

  - Physical distress or excessive workload such that the flight crew cannot be relied upon to perform their tasks accurately or completely; or

  - Serious of fatal injury to a relatively small number of the occupants other than the flight crew

Catastrophic: Failure conditions which would result in multiple fatalities, usually with the loss of the airplane.

In addition, each failure classification has a System Design Assurance Level (DAL) requirement associated with that particular classification. Table 3-1 below identifies the System DAL level based on FAR Part 25.1309 for each failure classification.

Table 3-1 – Design Assurance Levels (DALs)

| Failure Classification | Design Assurance Level (DAL) |
| --- | --- |
| Catastrophic | A |
| Hazardous | B |
| Major | C |
| Minor | D |
| No Effect | E |

The failure classifications, as well as the required DAL, for the applicable CNS systems identified in this report are listed in Table 3-2. These failure classifications are defined at the aircraft system level and not at the Line Replaceable Unit (LRU) level.

Table 3-2 – CNS Failure Classifications and Design Assurance Levels (DALs)

|  |  | Failure Classification |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CNS | System | Loss of Function | Undetected Erroneous Data | Radio DAL | Notes |
| COMMUNICATION |  |  |  |  |  |
| C | HF Comm | Minor | Minor | C | Applies to Voice and Data modes. Even though the most stringent failure classification is Minor, the HF radio DAL has traditionally been developed & certified to a DAL=C. |
| C | VHF Comm | Major | Major | C | Applies to Voice and Data modes |
| C | Inmarsat SATCOM Safety Applications | Minor | Major | D | Applies to Voice and Data modes. End-to-end data integrity is checked by higher-level application (e.g., FMS) |
| C | Iridium SATCOM Safety Applications | Minor | Major | D | Applies to Voice and Data modes. End-to-end data integrity is checked by higher-level application (e.g., FMS) |
| C | AeroMACS | Minor | Major | D | End-to-end data integrity is checked by higher-level application (e.g., FMS) |
| C | LDACS | Major | Major | C | Applies to Voice and Data modes |
| NAVIGATION |  |  |  |  |  |
| N | ILS Loc/Glideslope | Major Hazardous | Catastrophic | A | Loss of function is Major. Loss of all ILS deviation data during an autoland is a Hazardous effect. Catastrophic classification for erroneous data is driven by the CAT III autoland function. |
| N | VOR | Minor | Major | C |  |
| N | MB | Minor | Major | C |  |
| N | LRRA | Hazardous | Catastrophic | A | Loss of all radio altitude data during an autoland is a Hazardous effect. Catastrophic classification for erroneous data is driven by the CAT III autoland function. |
| N | GNSS | Major | Hazardous | A | Hazardous classification for erroneous data is driven by the Navigation function. DAL A is to address the safety objectives for RNP and to address GLS CAT III safety requirements. |
| N | GLS | Major Hazardous | Catastrophic | A | Loss of function is Major. Loss of all GLS deviation data during an autoland is a Hazardous effect. Catastrophic classification for erroneous data is driven by the CAT III autoland function. CAT I and CAT II is DAL B. |
| N | DME | Minor | Major | C |  |
| SURVEILLANCE |  |  |  |  |  |
| S | ATC Transponder | Minor | Major | B |  |
| S | ADS-B Out | Minor | Major | C | When developed as standalone function |
| S | TCAS/ACAS-X | Minor | Major | B | Software is required to meet DAL B per AC 20-151c, §2.3.8.4 |
| S | ADS-B In | Minor | Major | C | Most stringent FHA class of current ADS-B In applications is Major per AC 20-172B |

In addition, the failure classifications for combined failures of CNS functions need to be addressed. The failure classifications for combined failures of the CNS systems identified in this report are listed in Table 3-3.

Table 3-3 – CNS Combined Failure Classifications

| CNS | System Failures | Failure Classification | Notes |
| --- | --- | --- | --- |
| C | Loss of all Radio Communications | Major | Applies to loss of voice and data |
| C | Undetected Erroneous Radio Communications | Major | Applies to data only |
| N | Loss of all Radio Navigation | Major to Hazardous | Depends on where and which phase of flight the loss of function occurs |
| N | Undetected Erroneous of Radio Navigation | Hazardous to Catastrophic | Catastrophic during CAT III approach |
| S | Loss of all Surveillance Radios | Major |  |
| S | Undetected Erroneous of Surveillance Radios | Hazardous |  |
| C+N | Combined failure (loss or undetected erroneous) of radio Communications and radio Navigation | Catastrophic |  |
| C+S | Combined failure (loss or undetected erroneous) of radio Communications and Surveillance radios | Hazardous to Catastrophic |  |
| N+S | Combined failure (loss or undetected erroneous) of radio Navigation and Surveillance radios | Hazardous to Catastrophic |  |
| C+N+S | Combined loss or undetected erroneous failure of radio Communications + radio Navigation + Surveillance radios | Catastrophic | (from C+N) |

## Minimum Equipment List (MEL)/Dispatch Requirements

The Minimum Equipment List (MEL) identifies how long system/component repairs can be deferred. FAA MMEL Policy Letter 25, “MMEL and MEL Definitions” defines the following repair categories for all MELs approved under 14CFR Parts 91K, 121, 125, 129, 135, and 142:

Category A: Items in this category shall be repaired within the time interval specified in the Remarks column of the operator's approved MEL. For time intervals specified in “calendar days” or "flight days,” the day the malfunction was recorded in the aircraft maintenance record/logbook is excluded. For all other time intervals (flights, flight legs, cycles, hours, etc.), repair tracking begins at the point when the malfunction is deferred in accordance with the operator’s approved MEL.

Category B. Items in this category shall be repaired within three (3) consecutive calendar days (72 hours), excluding the day the malfunction was recorded in the aircraft maintenance record/logbook. For example, if it were recorded at 10 a.m. on January 26th, the three day interval would begin at midnight the 26th and end at midnight the 29th.

Category C. Items in this category shall be repaired within ten (10) consecutive calendar days (240 hours), excluding the day the malfunction was recorded in the aircraft maintenance record/logbook. For example, if it were recorded at 10 a.m. on January 26th, the 10 day interval would begin at midnight the 26th and end at midnight February 5th.

Category D. Items in this category shall be repaired within one hundred and twenty (120) consecutive calendar days (2880 hours), excluding the day the malfunction was recorded in the aircraft maintenance log and/or record.

The MEL categories for the applicable CNS systems identified in this report are listed in Table 3-4. Note that there are inter-relationships between equipment availabilities in order to be able to dispatch. For instance, dispatch is allowed when no GNSS systems are available if one DME is operative. This is an important consideration when considering integration of various radios on shared resources.

Note that MEL requirements may differ amongst regulators. Table 3-4 is representative of known, most stringent, MEL requirements at the time of publication.

Table 3-4 – CNS Minimum Equipment List (MEL)

| CNS | System | MEL Category | Number Installed | Number Required for Dispatch | Remarks or Exceptions |
| --- | --- | --- | --- | --- | --- |
| C | HF Comm - Voice | D | -- | -- | Any in excess of those required may be inoperative. |
|  |  | C | -- | 1 | May be inoperative while conducting operations that require two LRCS provided: a) Aircraft SATVOICE system operates normally, b) SATVOICE services are available as a LRCS over the intended route of flight, c) ICAO flight plan is updated to notify ATC of the communication status of aircraft, and d) Alternate procedures are established and used. |
| C | HF Comm – Data Link | C | 1 | 0 | May be inoperative provided alternate procedures are established and used. |
|  |  | D | 1 | 0 | May be inoperative provided procedures do not require its use. |
| C | VHF Comm - Voice | D | 3/2 | - | Any in excess of those required, and not powered by a standby bus, may be inoperative. |
| C | VHF Comm – Data Link | C | 1 | 0 | May be inoperative provided alternate procedures are established and used. |
|  |  | D | 1 | 0 | May be inoperative provided procedures do not require its use. |
| C | Inmarsat SATCOM | D | 2 | 1 |  |
|  |  | C | - | 0 | May be inoperative provided procedures do not require its use. |
| C | Iridium SATCOM | D | 2 | 1 |  |
|  |  | C | - | 0 | May be inoperative provided procedures do not require its use. |
| C | AeroMACS | C | - | 0 | May be inoperative provided procedures do not require its use. |
| C | LDACS | C | - | 0 | May be inoperative provided procedures do not require its use. |
| N | ILS Localizer/ Glideslope | C | 3/2 | 0 | Any in excess of those required may be inoperative provided approach minimums do not require their use. |
| N | VOR | C | 2 | 0 | Any in excess of those required may be inoperative. |
| N | MB | C | 1 | 0 | May be inoperative provided approach minimums do not require its use. |
| N | LRRA | C | 3/2 | 2/1 | One may be inoperative provided approach minimums or operating procedures do not require its use. |
| N | GNSS | C | 2 | 0 | May be inoperative provided alternate procedures are established and used. |
|  |  | C | 2 | 1 | One may be inoperative provided operations do not require its use. |
| N | GLS | D | 3/2 | 0 | May be inoperative provided approach minimums do not require its use. |
| N | DME | D | 2 | 0 | Any in excess of those required may be inoperative. |
| S | ATC Transponder | D | 2 | 1 | Any in excess of those required may be inoperative. |
| S | ADS-B Out | D | 2 | 0 | May be inoperative provided procedures do not require its use. |
| S | TCAS/ACAS-XA | C | 2 | 1 |  |
|  |  | B | 2/1 | 0 | May be inoperative provided: - System is deactivated, and - Enroute or approach procedures do not require its use. |
| S | ADS-B In | D | -- | 0 | May be inoperative provided enroute operations do not require its use. |

## Regulatory Requirements

The regulations governing CNS systems are predominately operational regulations (e.g., 14CFR Part 121). The regulations along with applicable FAA Technical Standard Orders (TSOs)/European TSOs, FAA Advisory Circulars, and EASA Acceptable Means of Compliance (AMCs) for the applicable CNS systems identified in this report are listed in Table 3-5.

Table 3-5 – CNS Systems Regulatory Material

The latest versions of the following documents apply.

| CNS | System | 14CFR Regulation(s) | TSO/ETSO | AC/AMC | Notes |
| --- | --- | --- | --- | --- | --- |
| C | HF Comm | 91.511, 121.99, 121.349, 121.351 | HFDL: C158() Voice: C170() | N/A |  |
| C | VHF Comm | 91.511, 121.99, 121.347, 121.349 | C128() VDL: C160() Voice: C169() | AC 20-67(), CS-ACNS () |  |
| C | Inmarsat SATCOM | 121.99, 121.351 | C159() | AC 20-150() |  |
| C | Iridium SATCOM | 121.99, 121.351 | C159() | AC 20-150() |  |
| C | AeroMACS | N/A | C207() |  |  |
| C | LDACS | TBD | TBD | TBD | Regulatory requirements have not yet been released. |
| N | ILS Loc/Glideslope | 121.349 | Loc: C36() G/S: C34() | AC 120-28(), AC 120-29() AC 120-118 AC 20-191 (draft) NPA 2018-06 (draft) |  |
| N | VOR | 121.349 | C40() | AC 20-138() |  |
| N | MB | 121.349 | C35() | N/A |  |
| N | LRRA | 121.354 | C87() | AC 120-118() |  |
| N | GNSS | 121.351 | L1 freq: C145(), C146() | AC 90-107(), AC 20-138(), CS-ACNS () |  |
| N | GLS | N/A | GBAS: C161() VDB: C162() | AC 120-118() AC 20-191 (draft) NPA 2018-06 (draft) |  |
| N | DME | 121.349 | C66() | AC 120-38() |  |
| S | ATC Transponder | 91.215, 121.356 | C112() | AC 20-151(), CS-ACNS () |  |
| S | ADS-B Out | 91.225, 91.227 | C166() | AC 20-165(), CS-ACNS () |  |
| S | TCAS/ACAS-X | 121.356 | TCAS: C119() ACAS-X: C219() | AC 20-151() | AC listed is for TCAS. The AC for ACAS-XA has not yet been released. |
| S | ADS-B In | N/A | Rcvr: C166() Apps: C195() | AC 20-172() |  |

## Radio Frequency (RF) Signal Characteristics

The radio signal and antenna characteristics for the CNS systems being evaluated are included in Tables 3-6 below.

Table 3-6 – CNS Antenna Characteristics

| CNS | System | Antenna Location | Antenna Type | Antenna Polarization |
| --- | --- | --- | --- | --- |
| C | HF Comm | Top | Omni | Vertical |
| C | VHF Comm | Top (1 or 2) Bot (1 or 2) | Omni | Vertical |
| C | Inmarsat SATCOM | Top | HGA: Directional-Steerable | Right-hand Circular |
| C | Inmarsat SATCOM | Top | Enhanced LGA | Right-hand Circular |
| C | Iridium SATCOM | Top | ALGA: Omni | Right-hand Circular |
| C | Iridium SATCOM | Top | LGA | Right-hand Circular |
| C | AeroMACS | Top | Omni | Vertical |
| C | LDACS | Top (1) or Bot (1) | Omni | Vertical |
| N | ILS Loc/Glideslope | AC Nose | Directional, FWD Looking | Horizontal |
| N | VOR | Top | Omni | Horizontal |
| N | MB | Bot | Omni | Horizontal |
| N | LRRA | Bot | Directional | Linear |
| N | GNSS | Top | Omni | Circular |
| N | VDB | Top | Omni | Horizontal |
| N | DME | Bot | Omni | Vertical |
| S | ADS-B Out ATC Transponder | Top (1) Bot (1) | Omni | Vertical |
| S | ADS-B In TCAS/ACAS-X | Top (1) Bot (1) | Top: Directional Bot: Omni/Directional | Vertical |

Table 3-7 – CNS Receiver Characteristics

| CNS | System | Rx Freq (MHz) | Rx Sensitivity (dBm) | Rx Modulation |
| --- | --- | --- | --- | --- |
| C | HF Comm | 2.8-24 | Voice: -101 HFDL: -101 | Voice: AM or SSB HFDL: PSK |
| C | VHF Comm | 118-137 | Voice: -87 Mode 2: -98 | Voice: AM Mode 0/A: AM-MSK Mode 2: D8PSK |
| C | Inmarsat SATCOM | 1525.0-1559.0 | Voice: Data: | Voice: Data: |
| C | Iridium SATCOM | 1616.0-1626.5 | Voice: Data: | Voice: Data: |
| C | AeroMACS | 5091-5150 | Note 1 | Note 1 |
| C | LDACS | 960-1164 | -104 | FDD using OFDM |
| N | ILS Loc/Glideslope | LOC: 108-112 GS: 328-336 | LOC: -86 GS: -76 | AM |
| N | VOR | 108-117.95 | -92 | AM/FM |
| N | MB | 75 | -60 | AM |
| N | LRRA | 4200-4400 | Note 2 | FMCW or Pulsed |
| N | GNSS | 1575.42 L1 L5 | -134 Varies by constellation | CDMA (for GPS, SBAS, GALILEO, and BEIDOU). FDMA (for GLONASS) |
| N | VDB | 108-118 | -87 | D8PSK |
| N | DME | 962-1213 | -83 | PPM Pulse Pair |
| S | ADS-B Out ATC Transponder | 1030 | -74 | PPM/DPSK |
| S | ADS-B In TCAS/ACAS-X | 1090 | -84 | PPM/PAM |

Note 1:  The AeroMACS receiver sensitivity (per ICAO Tech Manual (DOC 10044)):

Note 2:  There is no fixed minimum receiver sensitivity defined in the RTCA DO-155 or EUROCAE ED-30 MOPS, since it depends on the height above the ground and on the level of attenuation resulting from ground characteristics. Installations should meet the external loop loss characteristics in RTCA DO-155 Appendix B. Per DO-155, the external loop loss is the ratio of the available power entering the receiving antenna aperture to the power leaving the transmitter antenna aperture.

Table 3-8 – CNS Transmitter Characteristics

| CNS | System | Tx Freq (MHz) | Tx Power (dBm) | Tx Modulation |
| --- | --- | --- | --- | --- |
| C | HF Comm | 2.8-24 | 56 (400W) | Voice: SSB HFDL: PSK |
| C | VHF Comm | 118-137 | Voice/Mode 0/A: 44 (16W) Mode 2: (15W) | Voice: AM Mode 0/A: AM-MSK Mode 2: D8PSK |
| C | Inmarsat SATCOM | 1626.5-1660.5 | 42 (EIRP) |  |
| C | Iridium SATCOM | 1616.0-1626.5 | 42 (EIRP) |  |
| C | AeroMACS | 5091-5150 | Note 1 | QPSK |
| C | LDACS | 960-1164 | 42 per sub-carrier | FDD using OFDM |
| N | ILS Loc/Glideslope | n/a | n/a | n/a |
| N | VOR | n/a | n/a | n/a |
| N | MB | n/a | n/a | n/a |
| N | LRRA | 4200-4400 | Note 2 | FMCW or Pulsed |
| N | GNSS | n/a | n/a | n/a |
| N | VDB | n/a | N/A | N/A |
| N | DME | 960-1214 | 52 (ERP) | PPM, Pulse Pair |
| S | ADS-B Out ATC Transponder | 1090 | 53 (ERP) | PPM, PAM |
| S | TCAS/ACAS-X | 1030 | 52 (ERP) | PPM, PAM, DPSK |
| S | ADS-B In | n/a | n/a | n/a |

Note 1: The AeroMACS downlink transmit power (per RTCA DO-346 MOPS):

| Class | Max. Transmit Power (dBm) |
| --- | --- |
| 1 | 20 < PTx.max <= 23 |
| 2 | 23 < PTx.max <= 27 |
| 3 | 27 < PTx.max <= 30 |

Note 2:  There is no fixed minimum transmit power defined in the RTCA DO-155 or EUROCAE ED-30 MOPS since it depends on the height above the ground and on the level of attenuation resulting from ground characteristics. Installations should meet the external loop loss characteristics in RTCA DO-155 Appendix B. Per DO-155, the external loop loss is the ratio of the available power entering the receiving antenna aperture to the power leaving the transmitter antenna aperture.

## System Design Constraints

System Performance, installation, and airworthiness approval considerations should be the initial objectives that may help to formulate System design constraints. Communication, Navigation and Surveillance radios, implemented using a distributed architecture, must be compliant with applicable Minimum Operational Performance Standard (MOPS) as specified in (E)TSO requirements. It is recommended that TSO performances applicable to a particular communications, navigation, or surveillance radio system be provided by a single entity to avoid performance, integration and airworthiness approval challenges.

COMMENTARY

Multiple parties may provide components used in a single CNS radio function. However, this has the potential to increase the development cost and acquisition cost of the radio system when compared with the equivalent current federated radio system.

Each avionics radio system consists of both TSO and aircraft-specific (non-TSO) functionality. The aircraft-specific functionality may include the maintenance system interface, data loading system interface, analog or digital audio system interface, data link system interface, etc. To further reduce integration and testing costs, the same supplier/entity should supply both the Remote Radio Unit that hosts the TSO functionality and the aircraft specific software that may be hosted in a general-purpose computing platform that may not necessarily be provided by the same supplier. The physical interface between the Remote Radio Unit (RRU) and the general-purpose computing platform should be standardized.

The interface protocol between aircraft-specific applications and supplier-specific radio functions is expected to be defined by the airframe manufacturer and may be specific for that airframe type. To ensure seamless and cost efficient radio integration, each radio system supplier should control the data exchange between the functions hosted on the Remote Radio Unit and those hosted on the general-purpose computing platform – i.e., interface protocols may be unique to the radio system supplier.

Each distributed radio system consists of a Remote Radio Unit, interface communication lines, and software hosted on a general-purpose computing platform with an ARINC 653 operating system. The distribution of the radio system TSO and aircraft-specific (Non-TSO) functionality between the Remote Radio Unit and the software hosted on the general-purpose computing platform has been described above. In addition to those design constraints, the installation constraints, including maximum weight, form-fit (volume), and heat dissipation for each radio system specific Remote Radio Unit should be standardized. To minimize development and acquisition costs, all airframe manufacturers are expected to use the same physical layer and connectors for the interface communication lines between each specific Remote Radio Unit, antennas, and the general-purpose computing platform.

In addition, it is recommended that audio signals not be processed through the computing platform as it will introduce significant latencies. The audio signals should be managed directly between the Audio Management system and the Remote Radio Unit(s).

Figure 3-1 – Typical Architecture

Color coding in Figure 3-1 is as follows:

AIRFRAME Type Specific (BLUE & CAPITAL)

COMPUTING PLATFORM

AIRFRAME SPECIFIC APPLICATIONS

INTERFACE WITH AIRFRAME SPECIFIC RADIO FUNCTIONS

Radio Supplier Specific (Orange & Bold)

Remote Radio Unit TSO Software and Hardware

Data interface between Remote Radio Unit and radio functions in a central unit or a general-purpose computing platform

Antenna (in some cases)

Non-TSO airframe-specific functionality

Standardized by ARINC (Black & Italic)

Form, Fit, Function, and Installation of the Remote Radio Unit

Power supply to Remote Radio Unit

Interconnection connectors, cables, physical layer

Antenna interface

Operating System Interface (ARINC 653)

Control/Display Interface (ARINC 661)

Physical Network/Bus Interface to other avionics (e.g., ARINC 664, TSN, ARINC 429, or other)

Environmental requirements

Digital audio distribution architecture and digital audio interface

Software data loading, including associated configuration management support of Remote Radio Unit

Security support (key management, authentication, encryption, etc.)

An Example of Data flows for a Communication Radio is provided in Figure 3-2.

Figure 3-2 – Example of Data flows for a Communication Radio

## Digital Interface (RF-to-Processing) Constraints

The main constraints for the digital interface are the required data throughput and latency. This section focuses on throughput as latency is discussed in the following sections. Based on existing communication radio systems, the worst case for data throughput can be estimated. Table 3-9 provides a comparison between both a low-data-demanding communication system (e.g. VHF Comm) and a high-data-demanding communication system (e.g. Satcom). Data throughput will depend on the type of distributed architecture (Functional Allocation Options). The highest throughput will be necessary for an architecture with the Digital Signal Processing in the Central Unit, where all raw sampling data comes from the RRH (Type 0). Lower throughput is needed if the DSP is located in Remote Radio Unit (RRU) (Type 1a). The lowest throughput could be achieved in the RRU Type 2a configuration, where only application data is exchanged. The throughput numbers in Table 3-9 are best estimates only.

Table 3-9: Estimated Data Throughput for Typical Communication Radio Systems

|  | RRH Option (Signal Samples)NOTE 1 | RRU Type 1a (Processed Data) | RRU Type 2a (Application Data) |
| --- | --- | --- | --- |
| Low performance COM radio | ~ 2.5 Gbit/s | ~ 2.5 Mbit/s | ~ 600 kbit/s |
| High performance COM radio | ~ 8 Gbit/s | ~ 100 Mbit/s | ~ 1000 kbit/s |

NOTE 1:  The throughput estimates for the most demanding case are based on an assumption of: 4x sample rate of the system bandwidth and a sample size of 32 bits (16 bit I, 16 bit Q).

## System Processing Requirements

As mentioned above, TSO functionality (e.g., RF translation, waveform processing/modem, etc.) should be implemented in the Remote Radio Unit, whereas non-TSO “system functionality” may be implemented in a general-purpose computing platform. Utilizing a general-purpose computing platform avoids unnecessary duplication of system functions and isolates the Remote Radio Unit from aircraft-specific dependencies. Candidate system functions for the general-purpose computing platform include:

Aircraft system interfaces (I/O data concentration/abstraction)

Health/Maintenance support (fault logging/reporting, aircraft installation testing and troubleshooting)

Dataload (including associated configuration management support)

Datalink Router interface

Radio HMI support

Remote Radio Unit (RRU) mode control function and radio system configuration function

Upper layers (or application layer) of the radio system

Security support (key management, authentication, encryption, etc.)

## Audio Requirements

CNS radios should provide audible signals in compliance with the function provided.

### Audio Reference Documents

The latest version of the following documents should be considered in the design of audio system functions:

RTCA DO-214A: Audio Systems Characteristics and Minimum Operational Performance Standards for Aircraft Audio Systems and Equipment

EUROCAE ED-112A: Minimum Operational Performance Specifications for Crash Protected Airborne Recorder Systems

IETF RFC 3550 RTP: A Transport Protocol for Real-Time Applications

IEEE 1588-2008 Precision Time Protocol (PTPv2)

The AES67 audio networking interoperability standard includes a PTP profile compatible with SMPTE ST2059-2

### Bandwidth and Dynamic Range Considerations

The relationship of frequency bandwidth to dynamic range for communications/navigation and interphone interfaces is defined in RTCA 
DO-214A. As noted in RTCA DO-214A, full speech bandwidth extends from below 100 Hz to over 8 kHz for an adult male. However, the octave with the greatest energy (30%) is between 300 Hz to 600 Hz for an adult male, and 550 Hz to 
1100 Hz for an adult female. The majority of intelligibility is found between 1 kHz and 5 kHz. Reduction in bandwidth from 6 kHz is stated to reduce intelligibility in the presence of noise. Each of these factors contributes to the overall response of 
300 Hz to 6 kHz for the interphone interface. Therefore, radio systems are expected to provide audio response between 
300 Hz and 3 kHz.

Although RTCA DO-214A recommends no more than a -3 dB reduction in power at the pass-band edges, the overall response must remain within a 3 dB envelope. The use of extended frequency response elements is encouraged where practical. This minimizes the cumulative effect of cascaded components (microphone, audio system, headphones) on total system intelligibility. For example, a microphone, an audio system, and headphone, each having a 
300 Hz to 6 kHz (-3 dB) response, may produce a total system response of 600 Hz to 3 kHz (-3 dB) given a gradual roll-off in each component. The net effect is a reduction by four in the speech signal bandwidth and its energy.

Implementation of digital filtering can extend the bandwidth as recommended by RTCA DO-214A. Sample rate selection can balance the digital filter complexity versus consumption of network bandwidth. Typical sampling rates for 3 kHz radio paths and 6 kHz interphone paths are 8 kHz and 16 kHz, respectively. Maintaining dynamic range (minimum signal to maximum signal) is critical for maximizing intelligibility. Wiring practices in aircraft must maintain noise at levels 40dB below the desired speech level with the dynamic range of VHF and UHF audio Absolute and Differential Delay varying from 6 dB to greater than 40 dB. RTCA DO-214A requires the audio system to have greater than 50 dB to 60 dB of dynamic range, measured as ((S+N)/N) depending on the environmental category. To ensure system performance requirements are met, this minimum dynamic range must be maintained.

Choosing the appropriate quantization to balance the dynamic range versus consumption of network bandwidth is also important. The greater the dynamic range, the lower the audible background hiss when the system is idle (noise floor). The dynamic range requirement of 60 dB (suggested by RTCA DO-214A) may be achieved by using 11 bits or more per sample (using 2’s complement). Recommended audio quantization of 16 bits per sample is specified, yielding 90 dB of dynamic range.

### Absolute Delay

Absolute (Envelope) delays are specified in RTCA DO-214A. The absolute delay value must be defined due to the potential for adverse system affects like halted speech between operators. It is noted in RTCA DO-214A that the person speaking can become confused when the sidetone is delayed given sufficient amplitude. RTCA DO-214A minimizes the effects of latent sidetone by requiring less than 10 ms of absolute delay one way. Audio systems with greater latency in their digital audio transmission may use a local (internal) sidetone to meet the specification. However, careful consideration must be given to the microphone to radio/interphone delay and radio/interphone to headphone delay so as to not impede two-way communication. The maximum value for absolute delay (any microphone to any headset) using digital audio is 20 ms round trip for digital audio distribution. This value is based on industry studies of the acceptable tolerance of this delay by the speaker.

EUROCAE ED-112A places further restrictions on absolute delay as follows:

The delay in recording the flight crew audio signals from the time of reception at the microphones to the time of recording on the protected recording medium shall not exceed 250 milliseconds.

### Differential Delay

RTCA DO-214A mentions the possible case of two radios tuned to the same frequency and summed to the headphone. The resultant signal will be 6 dB louder if the signals have identical delay. However, a difference in delay may cause frequency nulls in the spectrum of the summed signal. This may reduce speech intelligibility due to the loss of formants. It may also result in the loss of pure tones such as 1020 Hz. Therefore, it is recommended to force the first null outside the system bandwidth by minimizing the differential delay.

RTCA DO-214A requires less than 9 dB of attenuation throughout the frequency bandwidth of the system when two identical signals are summed to the same output. The minimum differential delay to produce 9 dB of attenuation can be derived by analysis of the highest frequency in the pass-band. Only the 3 kHz pass-band edge for radio to headphone paths shall be considered since interphone to headphone paths have unique sources all the time. Of the 9 dB attenuation allowed, 3 dB is consumed by the -3 dB pass-band attenuation at 3 kHz, leaving 6 dB of attenuation due to differential delay. It requires only one third of a wavelength in time (period) of differential delay to achieve 6 dB of attenuation in the summed signal, or 114 μs at 
3 kHz.

Note: Considering next generation radio systems which may be capable of higher audio quality, a differential delay lower than 55us could be achieved which should aim for better audio quality.

Constant differences in the sampling intervals between asynchronous equipment due to sampling clock drift will result in timing errors. These timing errors may cause performance problems when the samples are too late or too early to properly replicate the audio signal. Accurate sample clocks are required to ensure adequate sampling performance.

### Architecture

This section describes several architecture variations with a few key implementation notes as described below. The architectures assume a shared digital network that carries all audio and control traffic.

Figure 3-3 shows a mixed scenario with two Audio Gateway functions that bridge between the digital audio domain and legacy CNS radio assets. Two digital only examples, a distributed VHF radio function and a distributed Satcom function, are shown below as examples where there is no analog to digital or digital to analog transfer taking place across the function boundary.

On the right side of the diagram, three flight deck user positions are shown, highlighting the digital to analog and analog to digital conversion at each station, synchronized via PTP with all other audio system assets.

Figure 3-3 – Audio Distribution Example

The definition of a standard full digital audio distribution architecture for next generation aircraft should be developed as part of a future effort.

### Transport Considerations

Digital audio places additional requirements on the audio distribution method due to the discrete nature of the digital audio sampling process. Digital transmission has the potential to affect audio quality in the event of timing errors. The relationship of these errors to the basic requirements described above is fundamental to establishing a digital audio distribution standard for the flight deck.

Real-time distribution of digital audio is often referred to as an audio stream. Ideally the digital audio stream is a continuous and steady flow of audio samples like the analog audio signal.

When samples are grouped together to create a network message payload, variable network transport delays may result in timing errors. These timing errors can result in payloads arriving too late to ensure adequate samples are available for replication. Proper buffering is required to ensure adequate streaming performance. These buffers are known as jitter buffers.

Network performance that can bound the nominal transport delay and maximum transport delay will provide predictable digital audio distribution performance. Bounding the maximum network delay allows the jitter buffer to be sized. The combination of the payload time interval, nominal network delay, and jitter buffer size is the audio distribution latency. Figure 3-4 Illustrates.

Figure 3-4 – Network Packet Latency

Additionally, the loss of a group of samples in a network message requires the need for special handling. Without special handling, loss of a single message may result in an audible “pop” or “click.” At the destination, each message must be in the same sequence as it was at the source. If the messages are out of order, the network interface must reorder them. A highly deterministic network is required to minimize the problems due to loss of samples or out-of-order samples.

This document recommends using RTP protocol without the following: QoS, SIP, and RTCP.

Audio streams are multicast when a stream from the publisher needs to be distributed to more than one subscriber.

There is a need to develop a standard specifying the payload and sampling rate of distributed digital audio signals. An example of this could be an RTP packet with 4ms of uncompressed audio, 16 bit, 16 kHz samples.

Maximum network latency should be considered to ensure meeting the 10ms one way absolute delay requirement.

### Synchronization Considerations

Synchronization between real-time publishers and subscribers is required to minimize audio noise due to dropped or added samples and to meet the RTCA 
DO-214A differential delay requirement.

There is a need to standardize these synchronization considerations. An example is the use of PTP per IEEE 1588-2008, where real-time publishers and subscribers synchronize their system and sample clocks to within 100µs of the master system time. This provides compliance with the RTCA DO-214A differential delay requirement and minimizes noise due to dropped/added samples which would otherwise occur due to buffer over/underruns in real-time. This is illustrated in 
Figure 3-5.

Figure 3-5 – Precision Time Protocol (PTP) Synchronization

The Audio System should have a method to compensate for clock differences between the publishers and subscribers of audio streams when the clock sync between audio equipment is lost.

The Audio System should provide a means to determine when clock sync between audio equipment is lost for the publishing audio equipment.

EUROCAE ED-112A, § I-2.1.8 places additional requirements on channel synchronization as follows:

The recordings for separate channels shall be made such that, when replayed, the relative time between channels can be deduced to better than 4 milliseconds irrespective of recording delay.

However, meeting the differential delay requirement is much more restrictive (the system must be synchronized to within 114 µs). Once synchronized, a system meeting the differential delay requirement will also meet the EUROCAE ED-112A requirement.

Note: Considering next generation radio systems which may be capable of higher audio quality, a differential delay lower than 55us could be achieved which should aim for better audio quality.

### Sidetone Considerations

Radio systems using local sidetone should ensure that the HF tune tone is heard in the local sidetone audio when present.

## Data Security

This section and its sub-sections provide data security aspects that should be addressed as part of the design of a distributed radio architecture.

### Security Assurance Level

Because the radio system has interfaces that are exposed to the outside world, system development activities should be subject to adequate security assurance in accordance with RTCA DO-326A/EUROCAE ED-203A. While the precise assurance levels are architecture-dependent, future standards may give minimum applicable assurance levels.

### Segregation

The proposed architecture may rely on different shared resources (e.g., networks, computing resources, and antennas). Shared resources are generally associated to risks of threat propagation and interference between different parts. To address such risks, the design should characterize the needs for segregating data flows and computing/processing resources. This also includes the segregation of data plane CNS traffic and aircraft-internal maintenance interfaces (e.g., for data loading, BITE and other centralized services).

### Radio Frequency Interference (RFI) Monitoring

Generally, wireless interfaces are directly exposed to RFI security threats. The system should put in place means of detecting and reporting such threats.

### Input Validation and Robustness

With any proposed architecture, the processing of CNS signals has several stages (antenna, RRU, aircraft-specific functionality, aircraft-specific applications). An adequate level of input validation / robustness to malformed inputs should be considered for each stage.

### Tamper Resistance

The design should contain requirements for the system to be resistant to unauthorized changes.

### Data Loading

Assuming that the system will include Loadable Software Parts, the design should include requirements on securing data loading.

### Centralized Security Functions

The system should use a centralized security services such as key management, crypto functions and security log collectors. If applicable standards are not yet available for such services, initiating such standards should be evaluated.

### Security Logging

As part of the system development, the following security logging aspects should be considered:

Provide designers of the system with a set of security logging guidelines

Establish a baseline for security related data that should be logged

Define the security and integrity of logs

Consider log file formats for interoperability

The focus should be on the generation, storage, transfer, and export of security logs. It does not consider the analysis of security logs which generally is performed on-ground.

# OBJECTIVES AND GOALS

## Introduction

This section describes the key goals and objectives for the design, implementation, and service of Distributed Radio architectures. These include design objectives, cost objectives, and the desire for interchangeability, reliability, and maintainability of avionics.

## Scope of This Section

The scope of this section is limited to describing the motivations and expected goals of Distributed Radio architectures in the context of total life-cycle costs, i.e., aircraft first cost, operational costs, cost of changes and additions, and maintenance cost. It attempts to categorize the benefits as they pertain to integration, fault tolerance, and modularity. It describes both direct and indirect benefits of applying Distributed Radio architectures to future aircraft systems.

## Distributed Radio Benefits

The Distributed Radio concept is expected to satisfy the objectives of the groups that traditionally participate in the definition of commercial aircraft avionics: the airlines as users and operators of the aircraft, the airframe manufacturer that design, build, and support commercial aircraft, and the equipment manufacturers that contribute to the innovative design, efficient production, and support of the components and subsystems

### Benefits to the Airlines

Reduced life-cycle cost through:

Increased operational performance, reduced empty weight, increased payload volume

Reduced unscheduled maintenance, reduced spares requirement

Simplified service life changes and additions to the avionics

### Benefits to the Airframe Manufacturer

Reduced first cost and cost of service life support of the aircraft through:

Reduced development, certification, and aircraft production costs

Reduced avionics weight and increased payload volume

Flexibility to efficiently meet customer requirements and to implement improvements

### Benefits to Equipment Manufacturers

Increased marketing opportunities of specialty components and subsystems through:

Increased market volume

Longer production runs

Flexibility to efficiently meet customer requirements

## Operational Objectives

There are several operational improvements that the airlines expect to achieve from the Distributed Radio concept. One prospect is to introduce new avionics maintenance philosophies which result in the ability to schedule maintenance actions so that no maintenance is required when the aircraft is away from a maintenance station. There is also an objective to reduce or eliminate unconfirmed removals through improved fault diagnostics.

Distributed Radio architectures are expected to allow the avionics equipment to take full advantage of technology changes and to expand efficiently. It is an objective to provide a capability to upgrade systems and to add new functions through on-board software loading of revised or all new software applications.

## Design Goals

Once the desired functional performance and operational safety are achieved, then the cost of ownership over the life of the aircraft is the primary criterion against which a system is judged. Cost of ownership should be used to trade off all other factors. The designer should provide avionics in which the sum of all contributing cost factors - development, amortization, materials, spares, weight, volume, operation, maintenance, test equipment, growth, etc. is minimized over the life of the aircraft. It is not acceptable to reduce one cost factor and neglect others. Particularly, it is not acceptable to favor first cost effects over continued life-cycle costs.

Airlines desire that cost of ownership models be developed and be maintained current for use in avionics upgrade programs and new development programs. Where cost of ownership models exist, users expect them to be employed in the cost analysis.

### Goals of Integration

The system design should make maximum use of shared resources to reduce resource duplication to a minimum. Such integration lowers the cost of ownership by reducing the acquisition cost, spares requirements, weight, and volume of the avionics equipment.

While hardware integration is desired, software functional independence is essential, and a certifiable method for partitioning these independent software elements from each other is necessary. Hardware integration should be limited by the desire to minimize complexity and spare unit costs.

The level of integration proposed in CNS distributed radio architectures suggests that software be on-board loadable to facilitate software updates to a function. This should be accomplished without removing from the airplane hardware relating to that function or any other hardware where the function is integrated.

### Goals of Fault Tolerance

#### Performance/Integrity

The approach to redundancy in Distributed Radio architecture can be viewed on two levels: functional level redundancy and component redundancy. Traditional systems achieve functional redundancy by duplicating the complete system. The network architecture of IMA could allow a hardware integrated approach and consequently reduce the number of components needed for a given level of function availability. In an IMA system, functional availability is ensured by providing multiple paths for the data from its source, to the processing required to the sink for the data (whether a display, audio, or other function). Component redundancy is necessary, but the emphasis is on a much greater use of fault containment techniques to allow other components in the system to continue functioning in the presence of failures.

The level and the physical method of redundancy used in each of the components should be totally transparent to the application software. Hardware should be designed independently of the application software so that changes in either do not affect each other. A detailed interface definition allows this approach to be possible. Thus, a standard interface allows competing or dissimilar designs to be used in different IMA processing modules without affecting the design of the application software. This will minimize the validation effort required and allow equipment manufacturers to have flexibility in the design of their equipment. Only the integrity of the integration has to be verified each time either the hardware or software changes. Separation of the environmental and application design allows the hardware and software to develop and mature at their own rates.

#### Scheduled Maintenance

The Distributed Radio architecture maintenance philosophy is built upon the desire for scheduled maintenance intervals. To achieve scheduled maintenance, it is necessary to establish fault containment areas throughout the architecture. Through this approach it is possible to quickly detect any failure and isolate it to a given fault containment area. A very high percentage of faults should be detected. Each of these fault containment areas should detect and annunciate the validity of its data to all users of that data. This approach allows the system to accurately report the status of its own health and enables users to achieve the maintenance goals that were previously unattainable. Therefore, it is a goal to make all avionics first failures transparent to the flight crew, annunciate all failures when interrogated by the maintenance crew, and allow maintenance to be scheduled at a convenient time.

COMMENTARY

Airline maintenance shops report that an excessive contributor to the cost of maintenance is unconfirmed (“No Fault Found”) removals. These costs are attributed to the time it takes to test and return good boxes to service and the cost of spares necessary to support this practice. Unnecessary equipment removal and handling is also known to cause maintenance-induced failures. The message from the users is to keep the good avionics on the airplane!

It is necessary to provide some level of resource redundancy to extend the Mean-Time Between Unscheduled Removal (MTBUR). The resource redundancy required to defer maintenance on a detected failure is dependent upon the reliability, accessibility, and the statistical probability of successfully operating for the deferral interval. Resource redundancy should be made available through secondary redundancy at the component level or it may be made available at a system level (as part of the aircraft architecture) by automatic reconfiguration.

It is recognized that desired reliability and extended maintenance interval are based upon individual airline maintenance philosophy. An individual avionic function should demonstrate graceful degradation. Furthermore, it is desired that the full avionic function continues to be available for at least a scheduled “A Check” maintenance interval. The overall system architecture design should contribute to the reliability goals for each avionics function.

## Equipment Packaging and Location

### Weight and Volume Considerations

Generally, users desire that avionics equipment be as light-weight and compact as possible. However, the desire for such equipment should not result in complex packaging designs that result in high-cost spares and compromise overall system integrity or life-cycle costs. The packaging designer is encouraged to work closely with the system designers and airframe manufacturers to evaluate the trade-offs of small, light-weight equipment.

### Location and Accessibility of Components

The trends toward the use of small, light-weight equipment with high Mean-Time Between Failures (MTBFs) and toward the use of a shared digital network on the aircraft provides the freedom to distribute equipment on the airplane in a variety of locations, thus eliminating the traditional Electrical and Electronics (E/E) bay. This will increase the aircraft’s useable payload volume. Equipment location should be determined based on a number of factors. It is the system integrator’s responsibility to analyze these factors before distributing equipment on the aircraft:

Function and operational performance

Environmental concerns

Physical access for maintenance

Remote access for software maintenance

Remote access for monitoring software logs

Maintenance philosophy

Integration with other systems

Growth potential and access for modifications

Lengths and number of wire runs and number of interconnects with source systems

Security threat concerns

Equipment access for maintenance purposes should minimize the impact to passengers and crew.

## Interchangeability

One of the continuing goals of airline users is interchangeability of avionics equipment, which is also desired by the airframe manufacturers. Interchangeability is necessary to achieve economies of scale, to distribute design and development costs, and to reduce the spares inventory. Ideally, interchangeability can be applied to any manufacturers’ components and between any two aircraft types and models. This has been achieved often enough in the past to prove that interchangeability is a worthy goal.

## Spares Provisioning

Spares provisioning is based on the maintenance plan developed for a particular airplane and the extent that they are adapted to meet the needs of an individual airline. The objective of the users is to procure the fewest number of spares without jeopardizing aircraft dispatchability.

The goal is to employ fault tolerance to defer maintenance actions and reduce the number of spares required.

## Distributed Radio Implementation

As a follow-up to this document, the industry may establish ARINC Standards for individual hardware LRUs and associated interfaces. This will allow individual LRUs to be installed in multiple locations on the airplane and on multiple airplane types. Standardization is expected to result in interchangeability between various manufacturers products.

The airlines desire reduced cost of ownership through improvements in aircraft operational areas that affect day-to-day costs. Notably, higher dispatch availability is expected using high-MTBF equipment. The application of fault tolerant design is expected to reduce the number of unplanned maintenance actions. Improved diagnostic capability should make flight line maintenance actions swift when failures do occur.

For some equipment, the low frequency of maintenance action may result in airlines finding it economically practical to defer component maintenance action to the equipment suppliers. This situation results in closer relationships between the user and the supplier. Furthermore, users may choose to employ the supplier to perform other enhancements or modifications to equipment.

Airline users expect the airframe manufacturer to assume the responsibility for system integration. This includes installation of equipment, communication between subsystems, and ensuring fault data integrity to the LRU or Line Replaceable Module (LRM) level.

In a CNS distributed radio architecture implementation, a particular function may be divided into smaller pieces resulting in the potential for more suppliers to contribute to the complete function. However, as previously stated in Section 3.5, System Design Constraints, it is recommended for CNS distributed radio architectures that TSO performances applicable to a particular communications, navigation, or surveillance radio system be provided by a single entity to avoid performance, integration, and airworthiness approval challenges. System integrators can be negatively affected where equipment from various suppliers is integrated to perform a specific radio function. Care should be taken to ensure that problems identified during the integration process and throughout the product life are handled in an expedient and cost effective manner.

In the Distributed Radio architecture concept, there is need to prove the operational compatibility for all equipment used in the design. Airframe manufacturers, as system integrators, will need to work closely with avionics suppliers to assure this compatibility.

In the software area, the IMA concept will introduce a standardized operating system interface based on ARINC 653. This will add significant lines of code and associated complexity. Likewise, remote units are evolving from pure hardware implementations to software defined radios. This enables suppliers to competitively supply software to implement different avionics applications. It is expected that users, hardware suppliers, and software suppliers will establish closer relationships as a result.

## Distributed Radios in Flight Simulators

Flight simulators as well as maintenance trainers are now recognized as an essential part of the aviation industry. Airlines have become more and more dependent upon such simulators for flight crew and maintenance crew training. Airlines typically require these simulators to be available as early as possible to allow for crew training prior to equipment introduction into revenue service.

The functions required to support the airlines training needs are specified in the latest version of ARINC Report 610: Guidance for Design and Integration of Aircraft Avionics Equipment in Flight Simulators.

As a general rule, avionics designers should develop hardware/software architectures and algorithms that are compatible with the simulator functions. Each software application should be capable of being individually controlled and should also report its operational status. A keep-alive signal may also be transmitted continuously to the software to indicate the installation in a simulator.

# Requirements on Supporting Technologies

## Introduction

The Distributed Radio architecture will depend on the application of established supporting technologies which can be applied to form complete systems. This section describes those elements which form a fundamental part of the architecture.

## ARINC 664 – Avionics Full-Duplex Switched Ethernet Network

Since the target for the Distributed Radio architecture are new aircraft types, the general purpose computing platforms could use ARINC 664 Part 7 as the data network used for intra-avionics communications. Therefore, Remote Radio Units could use the ARINC 664 interface standard. The details of the ARINC 664 data network are documented in ARINC Specification 664: Aircraft Data Network, Part 7, Avionics Full-Duplex Switched Ethernet Network. The use of ARINC 664 Part 7, current version, or future evolution of that standard, needs to be considered.

Alternatively, to support high bandwidth needs, including audio data flows, the use of a Time Sensitive Network (TSN) could be considered.

The treatment of digital audio is discussed in Section 3.8 of this document.

## ARINC 653 – Application Software Interface

A standardized application software environment is part of the Distributed Radio concept. It is envisioned that some of the system processing is accomplished with one or more software applications hosted on an IMA platform. ARINC Specification 653 defines an interface standard between the operating system software and the system’s application software. ARINC 653 defines the communication services and memory management facilities expected to be used with Distributed Radios.

The placement of software applications is distributed among the IMA’s network of processors. There are several applications hosted on each processor. These applications may originate from different avionics sources and be integrated into the selected implementation of the core processing hardware. It is necessary to ensure reliable software partitioning to create “brick walls” between applications, especially where these applications may be different levels of software criticality. Common services such as codecs, crypto, communalized interfaces are TBD.

Reliability and availability goals may be satisfied by hosting software applications on multiple IMA platforms capable of self-monitoring and fault tolerance.

## ARINC 624 – Onboard Maintenance System

ARINC Report 624 is a design guide for On-Board Maintenance Systems (OMS). The OMS design guide discusses a variety of maintenance concepts such as Built-In Test Equipment (BITE), BITE access, and Aircraft Conditioning and Monitoring Systems (ACMS). The document recommends an English-based user interface, non-volatile BITE storage, and Onboard Maintenance Documentation (OMD).

## ARINC 661 – Cockpit Display System (CDS) Interfaces

ARINC Specification 661 defines an interface protocol intended to facilitate communication between the Cockpit Display System (CDS) and user equipment. ARINC 661 defines two external interfaces between the CDS and the aircraft systems. The first is the interface between the avionics equipment (user systems) and the display system graphics generators. The second is a means by which symbology and its related behavior are defined. A User Application is defined as a system that transmits data to the CDS, which in turn can be displayed as visual graphical information to the flight deck crew. A User Application can also include software or hardware that receives input from interactive graphics managed by the CDS.

Some examples of a User Application are:

- Cockpit Display of Traffic (CDTI) - display of the surrounding ADS-B and ATC Transponder equipped aircraft

- Traffic List display and controls

- In-Trail Procedure (ITP) display and controls

## ARINC 8xx – Radio Packaging Standard

Packaging standards defined by ARINC Specification 836A: Cabin Standard Enclosures were discussed as a possible starting point for the development of radio packaging standards. ARINC 836A is intended for cabin installation environments that are relatively benign compared to radio installation locations.

## Fiber Optic Interfaces

It should be considered if fiber optic connections could be appropriate physical interfaces for Remote Radio Units (RRUs). There are advantages (fiber links are light-weight and immune to RF interference) and drawbacks (higher risks of installation issues). Fiber optic hardware definitions continue to be standardized. ARINC Specification 801 provides standardization of a fiber optic interconnect assembly, which is composed of a connector, fiber optic cable, and fiber optic termini. In addition, ARINC Specification 845 defines a fiber optic Expanded Beam (EB) termini for the air transport industry with the goal to avoid the proliferation of different designs of termini to serve the same functions on different aircraft models.

## Related Documents

The latest version of the following documents apply:

EUROCAE ED-14: Environmental Conditions and Test Procedures for Airborne Equipment (latest revision)

EUROCAE ED-12: Software Considerations in Airborne Systems and Equipment Certification (latest revision)

RTCA DO-160: Environmental Conditions and Test Procedures for Airborne Equipment

RTCA DO-178: Software Considerations in Airborne Systems and Equipment Certification

RTCA DO-254: Design Assurance Guidance for Airborne Electronic Hardware

RTCA DO-297: {IMA}

RTCA DO-326: Airworthiness Security Process Specification

# CNS DISTRIBUTED RADIO ARCHITECTURES

## Introduction

This section defines some terms to allow a common vocabulary to be used when describing Distributed Radio system architectures.

The physical architecture is targeted, in particular the allocation of functionality between central and remote areas, with a view of increasing the use of remote locations in order to reduce overall system Size, Weight, and Power (SWaP).

This architecture section looks at various architectures and makes some assessments of the potential advantages and disadvantages that each bring.

### Architecture Scope

There are a large and growing number of CNS systems on a modern airplane, each of which should be considered for the potential benefits of the application of a distributed architecture. Figure 6-1 describes the general situation and shows two dimensions that need to be assessed to define the scope of this study. A further dimension is time, which may be used to separate the introduction, and potentially specification, of distributed radio architectures for each new aircraft type.

Furthermore, discussions are underway that may lead to decisions on whether to allow individual systems, or groups of systems, to have different architectures or be implemented independently of each other.

Figure 6-1 – Potential Scope of Distributed Architecture for CNS Systems

## Distributed Architecture [Functional Allocation Options and Proposed Nomenclature]

It is the intention within this section to discuss the various options for locating parts of the functionality of each radio system and to introduce some nomenclature that can be used to refer to them.

For the purpose of this discussion a generic radio has been broken into five sections of functionality which apply to both transmit and receive paths and are described below. It is likely that not all radios will fall neatly into this categorization; however, the general principles will still apply. In particular there are implications on the interfacing requirements between units that are determined by where these functions are located and are important when considering a distributed architecture.

This is typically a power amplifier stage in the transmit path and a Low Noise Amplifier (LNA) or first stage of receiver to control noise figure. Also, any RF functionality between these and the antenna(s) such as filtering, switching, and duplexing.

This stage includes up/down frequency conversion and the analogue signal conditioning prior to analogue to digital conversion and conditioning after digital to analogue conversion. Critically, this stage includes any analogue to digital conversion and consequently the interface to following receive and preceding transmit stage is digital. This will have to operate at the sampling rate with consequently higher bandwidth.

A significant proportion of modern radios are implemented in the digital domain and this section would cover the Digital Signal Processing (DSP) processors and supporting systems for these functions. The interface is now only passing data at the information bandwidth with a small overhead for line and network protocols.

Many radio systems include complex protocols to deal with lossy lower layers and to enable multiple users to interact on a network. These are typically implemented using a General Purpose Processor (GPP).

This is the physical and software interfaces from the radio function to the aircraft that enables the radio to be controlled and data transferred.

### Central and Remote Locations

The term “central” is used in this document to refer to units located in a central location on the aircraft, typically specifically designed to accommodate electronics enclosures and which provides a degree of environmental protection. Such an area would be the EE bay where racking providing cooling air is also available.

The term “remote” is used in this document to refer to a number of potential locations that are generally around the periphery of the aircraft such that they are close to the antenna. It is assumed that these will be in pressurized areas but that there will not be any specific racking or cooling provided. Consequently, the SWaP of these units is important. Likely locations are the crown area (green) and the triangle area (red) as shown in Figure 6-2.

Figure 6-2 – Proposed Remote Locations

### Remote Unit Options – Passive Antenna

Each row of Figure 6-3 below represents a different allocation of the functionally to the physical architecture where a simple passive antenna is used. Specifically, between units located in a central location, a remote location assumed to be near a corresponding antenna, or the antenna on the outside of the aircraft.

Existing radio architectures are federated and broadly have the functionality allocated to the physical architecture represented in the first row of Figure 6-3. Here, all of the functionality is contained in a unit located in a central environmentally controlled location in the aircraft, usually the EE bay, and is connected to an antenna mounted externally via a low loss coaxial cable. The term “collocated” is used, since all of the functionality is located in one area. In this case there is no need for a remote unit, other than an antenna, as all functionality is contained in the central unit(s).

A collocated allocation results in the need for long runs of coaxial cable between the central area and the external antenna which must have low loss. The consequence is typically a requirement for long thick cables (high mass) which constitutes a significant proportion of the overall system mass. This is compounded by the need for more radio frequency power output from the transmitter to compensate for cable loss.

The second row shows a different allocation to the physical architecture, which has been introduced for some communication radios e.g., L-Band Satcom. Here the location of the final RF stages for power amplification and low noise amplification (including diplexing) are located near the antenna in a remote location to reduce losses to and from the antenna. This reduces the transmitter power output required and combined with an LNA, to control the noise figure, allows higher loss coaxial cabling to be used for the long runs between the remote and central unit(s). This arrangement is referred to as an analog split because the interface between the central and remote units will remain analog.

The third row provides additional functionality to the remote unit, critically the digital to analog conversation stage, thus allowing the use of a high speed point-to-point digital interconnect between the remote and central units carrying sample rate data. This allocation is therefore referred to as a digital split. This architecture is common in mobile phone base station designs where the remote unit is referred to as a Remote Radio Head. While this architecture has advantages, it is not considered to be fully distributed since it requires a sample rate digital interface which may prevent its application where remote units are connected using digital network interfaces.

In the final two rows, most functionality is provided in the remote unit allowing the use of digital networks to connect the remote and central units since the bandwidth is reduced close to the data rate and interface stability and latency is less critical. These two allocations are considered to be fully distributed. To distinguish the remote unit in these options, from that in the previous digital split architecture, the unit is referred to as a Remote Radio Unit (RRU).

Figure 6-3 – Progression from Federated to Distributed (Passive Antenna)

### Remote Unit Options – Active Antenna

If active antenna configurations are considered, then further possibilities for the allocation of the functionality between the RRU and antenna exist. Figure 6-4 shows the potential options for Type 1 RRUs intended for use with active antennas.

Figure 6-4 – Further Distribution – Type 1 RRU Options (Active Antenna)

Figure 6-5 shows the equivalent active antenna options for Type 2 RRUs. However, it should be noted that one row has been indicated as unlikely to provide significant benefit due to the limited functionality that would be incorporated into the RRU type that it generates.

Figure 6-5 – Further Distribution – Type 2 RRU Options (Active Antenna)

The final row in Figure 6-4 and Figure 6-5 above shows the situation where a remote unit is not required, and all the functionality is provided in the antenna.

### RRU Type Definition

Remote Radio Unit (RRU) types are classified by the amount of radio functionality provided in a centralized unit(s) versus the RRU(s) and the specific interface requirements between the RRU(s) and the centralized unit(s)

RRU Type 0 –Digital signal processing is performed in the centralized unit(s) and may require a high-speed point-to-point digital interface.

RRU Type 1 – All functionality apart from Radio Interface Management and Line/Network Protocols is located in RRUs.

RRU Type 2 – This provides the most extensive amount of functionality located in a remote location leaving only the Radio Interface Management within a centralized unit.

A second character is appended to detail the allocation of the functionality between the RRU and antenna.

When a RRU Type 1 or RRU Type 2 unit is intended to operate with a passive antenna it has an “a” suffix which indicates that no active signal path functionality is migrated into the antenna.

When the RRU is designed as part of a system that incorporates an active antenna, and where some functionality is located in the antenna, the second alpha character denotes how much functionality is located in active antenna systems. The figures above show the RRUs labeled accordingly.

RRU Type Xa – Passive antenna only

RRU Type Xb – Power amplifier (and LNA) located within antenna 
sub-system

RRU Type Xc – Analog Signal Processing additionally located in antenna sub-system

RRU Type Xd – Digital Signal Processing additionally located in antenna 
sub-system

Note:	The utility of RRU Type Xd appears to be very limited and consequently could be removed.

Where antennas contain powered devices that do not form part of the RF signal path they are considered to be “passive” in the nomenclature defined above.

### Antenna Type Definition

Antennas are classified by the amount of functionality they contain. This list is arranged such that a Type A antenna will correspond to a Type Xa RRU.

Antenna Type A – Passive Antenna

Antenna Type B – An active antenna which contains Tx power amplification, Rx LNA and any RF switching, or duplexing required. The signal interface to/from the antenna are low power RF with control functionality provided by a separate physical interface or multiplexed over coaxial cable.

Antenna Type C - An active antenna, which in addition to Type B, also contains the radio analog signal processing and digital-to-analog conversion such that a digital interface is required which is capable of transferring sample rate data and any corresponding timing constraints.

Antenna Type D – An active antenna, which in addition to Type C, also contains any digital signal processing required for the signal path. The digital interface is required to pass data at the information bit rate with a corresponding lower bandwidth and more relaxed timing.

Antenna Type E – This provides the most extensive amount of functionality located in the antenna (external to the aircraft) such than no RRU is required and the antenna itself connects directly to a centrally located control unit.

### Multiple Bearer RRU Options

It is possible to include two, or possibly more, radios within a single remote enclosure to create a multiple bearer (i.e., channel/radio) RRU.

Multiple instances of the same radio type (multiple channel), or different radio types (multiple radio) could be considered.

When considering multiple radios there are many possible ways of selecting the radios to be grouped, for example:

Identical function (e.g., multiple instances of a VHF radio)

Related function (e.g., within Communications, rather than Navigation or Surveillance)

Frequency bands close (e.g., Iridium and Inmarsat L-Band satcom)

Similar or identical DAL (e.g., DAL C)

When more than one radio is included within an RRU there is the potential to share a single antenna. Clearly, there are a number of further considerations to be taken into account when determining if this is viable. These considerations are discussed in other sections of this report. This section focusses on the architecture only.

Clearly, various combinations are possible. The option represented in Figure 6-6 is one of many possible combinations.

Figure 6-6 – Example of Dual Channel or Dual Radio RRU (Separate Antenna)

A further option is to combine multiple radios and share a single antenna as shown in Figure 6-7 below,

Figure 6-7 – Example of Dual Channel or Dual Radio RRU (Shared Antenna)

### Consistency of Architectures Across Breadth of CNS

There is no reason, in principle, to constrain the architectures employed by each RRU to be consistent across all of the CNS systems. It is likely that permitting variation may provide a more optimized solution overall.

In Figure 6-8 below, mixed RRU architectures are shown that are deployed both between CNS and within each unit TBD.

Figure 6-8 – Example of Mixed Architectures across CNS Systems

It is proposed that the CU function be implemented on an IMA like platform. This will enable the support of more than one radio function. While this can reduce the SWaP of the CNS system(s), it does potentially introduce a growth in the design assurance level required by any shared component – since the DALs assigned to current federated equipment are based on the assumption that each federated equipment operates independently.

## Legacy Radio Architectures (Communications)

This section shows the existing Communication architectures in a consistent style to that being used for discussion of future distributed radio architectures.

### HF Federated

Figure 6-9 – Federated HF Radio Example

### VHF Federated

Figure 6-10 – Federated VHF Radio Example

Figure 6-11 – Dual VHF Radio Example

### L-Band Satcom (Inmarsat) Federated (Compact Configurations)

Figure 6-12 – L-Band Satcom Example 1

### L-Band Satcom (Iridium) Federated

Figure 6-13 – L-Band Satcom Example 2

### LDACS

No physical architecture defined (standardized) at this time.

### AeroMACS

No physical architecture defined (standardized) at this time.

## Legacy Radio Architectures (Navigation)

This section shows the existing Navigation architectures in a consistent style to that being used for discussion of future distributed radio architectures.

### ILS (Localizer/Glideslope) Federated

Figure 6-14 – Federated ILS Example

### VOR Federated

Figure 6-15 – Federated VOR Example

### Marker Beacon (MB) Federated

Figure 6-16 – Federated MB Example

### LRRA Federated

Figure 6-17 – Federated LRRA Example

### GNSS Federated

Figure 6-18 – Federated GNSS Example

{Editor’s Note: For MMR and GNSS Rcvr config, move AMP block into the Active Antenna. Also, show A743 Brick at a remote location}.

### GLS/VDB Federated

Figure 6-19 – Federated GLS/VDB Example

### DME Federated

Figure 6-20 – Federated DME Example

{Editor’s Note: Reference A709}.

## Legacy Radio Architectures (Surveillance)

This section shows the existing Surveillance architectures in a consistent style to that being used for discussion of future distributed radio architectures.

### ATC Transponder and ADS-B Out

Figure 6-21 – Federated ATC Transponder and ADS-B Out Example

### TCAS/ACAS-XA and ADS-B In

Figure 6-23 – Federated TCAS/ACAS-XA and ADS-B In Example

## Hybrid Distributed Architecture

It is likely that the introduction of a fully distributed architecture with new interfaces into other aircraft systems will require a new aircraft type and, even then, will lack the necessary pedigree from in- service operation to make this possible.

A potential route to obtaining the necessary pedigree would be to accommodate a mix of distributed radios with the traditional federated radios. Further, those systems which are distributed may make use of a centralized unit that provides interfaces to the aircraft systems which are compatible with the existing federated standards.

The examples below are for L-Band Satcom, but the principle can easily be applied to any other radio system.

Figure 6-25 – Example Compact Satcom (ARINC 781)

Figure 6-26 – Hybrid Architecture Example

# REMOTE RADIO UNITS (RRU)

## Installation

Remote Radio Units (RRUs) are envisioned to be mounted between the fuselage and the cabin panels of the aircraft, or in other distributed installation locations outside the avionics equipment bay. In these cases, the traditional packaging standards defined by ARINC Specification 600 may not be appropriate. For example, ARINC 600 forced-air cooling will not be available. The equipment may also be subjected to condensing moisture conditions which will restrict the use of cooling holes.

Internal RRU fans could be used to provide air circulation for cooling. However, internal cooling fans are undesirable from a noise and reliability perspective and would need to be assessed. EMI performance needs may vary by fuselage types and equipment locations, especially when considering that the electronics may be mounted in close proximity to its own antenna, or the antenna(s) of other systems. Radio packaging concepts are shown in Section 7.4.

Figure 7-1 illustrates potential areas where Remote Radio Units could be installed.

Figure 7-1 – Potential Remote Radio Unit Location Areas

## Power

Remote Radio Units should use 28 Vdc input power.

## Environmental Requirements (In-Work)

Table 7-1 – RRU Environmental Requirements

| Environmental Requirements | ED-14/ DO-160 Section | Category | Applicability Criteria | Airframe Requirements |
| --- | --- | --- | --- | --- |
| Temperature & Steady State Altitude | 4 | A1 | Flight deck/cabin (pressurized and controlled temperature areas, not including crown) | (1) |
|  |  | A2 | Any other pressurized and partially controlled temperature areas (e.g., cargo areas) |  |
| Altitude: Decompression / Overpressure | 4 | A1 | Flight deck or cabin |  |
|  |  | A2 | Any other pressurized section |  |
| Temperature Variation | 5 | C | Flight deck or cabin (temperature controlled areas) |  |
|  |  | B | Any other internal area of the aircraft (non-temperature controlled or partially temperature controlled areas) |  |
| Humidity | 6 | A | Pressurized area |  |
| Operational Shock | 7 | A / D / B / E | All zones | (2) |
| Crash Safety Impulse | 7 | B, E | Equipment or part thereof that, if detaches from installation in the event of a crash, could cause injury to people or damage to the following equipment: Equipment belonging to fire detection or extinguishing system Emergency evacuation equipment Equipment belonging to fuel, hydraulic, or oxygen systems, whose leakage could fuel a fire during a crash event | (2) |
| Operational Vibration | 8 | R | For equipment for which resistance to the effects of long exposure to vibration is required | (2) |
|  |  | H | High-level, short duration transient vibration encountered during engine fan blade loss. | (2) |
| Explosion Proofness | 9 | N/A | Not required for any CNS system |  |
| Waterproofness | 10 | R | Pressurized areas close to a door or hatch that can be exposed to the outside environment |  |
|  |  | W | Internal areas where condensation occurs |  |
| Fluids Susceptibility | 11 | F | All areas | (3) |
| Sand & Dust | 12 | S | Pressurized areas close to a door or hatch that can be exposed to the outside environment and not protected (e.g., by a lining) |  |
|  |  | D | Other equipment located in pressurized areas |  |
| Fungus Resistance | 13 | F | All areas |  |
| Salt Fog | 14 | T | Category T: 2 cycles of 48 hours each. This is the most severe category [for RRU installed in pressurized areas close to a door or hatch that can be exposed to the outside environment and not protected (e.g., by a lining)] |  |
| Magnetic Effects | 15 | Z, A, B, or C | Category is dependent on equipment location and distance from magnetic compass: Z: D ≤ 0.3m A: 0.3m < D ≤ 1m B: 1m < D ≤ 3m C: D ≥ 3 m Equipment located close to the flight deck could affect the standby magnetic compass. |  |
| Power Input | 16 | A, B, or Z + sub-category R (ripple test) | Test category is dependent upon the type of 28 Vdc aircraft power source. |  |
| Voltage Spike | 17 | A |  |  |
| Audio Frequency Conducted Susceptibility – Power Inputs | 18 | R, B, or Z | Test category is dependent upon the type of 28 Vdc aircraft power source. |  |
| Induced Signal Susceptibility | 19 | C() or Z() depending upon the installation and the aircraft’s primary power source | CC is for installations with long wire runs or minimum separation. | (6) |
| Radio Frequency Susceptibility | 20 | W / R | EMH Category A, EMH Category B, C, D | (6) |
| Radio Frequency Emissions | 21 | L, M,H, P or Q | Category is dependent on equipment location. | (6) |
| Lightning Indirect Effects | 22 | Per DO-160G §22.3 |  | (6) |
| Lightning Direct Effects | 23 | N/A |  |  |
| Icing | 24 | N/A |  |  |
| Electrostatic Discharge | 25 | A |  |  |
| Flammability | 26 | C |  | (4) |
| Smoke & Toxicity | --N/A | N/A |  | (5) |

Notes:

Ground Survival High Temperature for equipment in Crown area is +95 °C.

For equipment + fixture; each airframe manufacturer has specific vibration and acceleration requirements (including PRA: Nose Wheel Imbalance, Fan Blade Out, and Sustained Engine Imbalance).

Each airframe manufacturer has specific list of fluids and temperatures.

Each airframe manufacturer has specific requirements: RTCA DO-160 Section 26 could be an acceptable means.

Each airframe manufacturer has specific requirements.

Each airframe manufacturer has specific requirements. Test levels vary depending on safety objectives and criticality (i.e., DAL), as well as Electromagnetic Hazard category of the equipment.

## Form Factors/Packaging

General requirements for the packaging and installation of remote radio units presents a challenge when compared to traditional avionics located in the equipment bay. In order to place the radio electronics near the antenna, the installation must consider the volume of space available, structural mounting performance, thermal performance, EMI, and a change in the maintenance concept of the installation.

When mounted between the fuselage and the cabin panels of the aircraft, or in other distributed installation locations outside the avionics equipment bay, ARINC 600 forced-air cooling is not available. The equipment may also be subjected to condensing moisture conditions which restricts the use of cooling holes. Additionally, internal RRU fans could be used to provide air circulation for cooling. However, internal cooling fans are undesirable from a noise and reliability perspective and would need to be assessed. EMI performance needs may vary by fuselage types and positions, especially when considering that the electronics may be mounted in close proximity to its own antenna, or the antenna(s) of other systems.

By way of an example to address design requirements and promote a standard approach to packaging, ARINC Specification 836A: Cabin Standard Enclosures provides some level of standardization. The reader should keep in mind that ARINC 836A was prepared to support cabin systems. ARINC 836A provides general guidelines for design of the enclosure.

Figure 7-2 – ARINC 836A Type I Enclosure Example

The benefits of ARINC 836A Type I packaging include standardization of mounting footprint and enclosure volume. Thermal dissipation is achieved through conduction to the attachment points, or convection into the surrounding air. Though increase in temperatures within the cabin and a lack of forced-air cooling may reduce predicted reliability when compared to electronics installed in the avionics equipment bay, it has been determined analytically and through prototype testing that relatively power-dense avionics can be sufficiently cooled in these conditions when designed for conductive thermal dissipation.

When considering downsides, ARINC 836A Type I packaging offers full freedom for connector selection and position, which can create challenges when desiring a standardized connector solution.

ARINC 836A defines a Type II small form-factor enclosure tailored to the cabin environment. It includes a tray installation/ejection mechanism that requires no tools. This form factor is referred to as a Miniature Module.

Figure 7-3 – Design Progress using ARINC 836A as an Example

ARINC 836A Type II packaging is not well-suited for thermal dissipation. The specification includes very little conductive dissipation, while convective dissipation is limited by the enclosure volume and installation. The specification also offers only four form-factors, which may not be sufficient when considering the variations between radios in the CNS suite.

In order to meet requirements for RF-based CNS systems, design concepts similar to that provided in ARINC 836A show promise, but it need refinement to meet packaging considerations for the Distributed Radio concept.

## Connectors

Note that each Airframe manufacturer has a list of connectors that are allowed to be used. This list is used to limit the variability and the number of different connector part numbers.

While ARINC 836A Type I provides guidelines for connector position, it does not specify connector size, shape, or composition. This may meet the needs of the radio designer, but it is undesirable for industry standardization. ARINC 836A Type II specifies the connector shell size and position, however, the modular nature of ARINC 836A Type II enclosures limits the positioning of connectors to each quadrant. This is opposed to a centerline position that adjusts with the size of the box, similar to ARINC 600. While the connector series is addressed by EN-4165, connector suppliers will need to develop standardized plugs and backshells to meet the needs of avionics at the same level of detail provided by ARINC 600. Standard plug types and standard layouts are not specified in ARINC 836A.

Circular connectors, such as the Mil-DTL-38999 series, are expected to meet the functional requirements and the environmental conditions.

At present, collaboration between the CNS system supplier and the airframe manufacturer is necessary to select a connector that meets specific system requirements.

Connector standards that meet the needs of this Distributed Radio concept are TBD.

## Interfaces

A shared digital aircraft network is the recommended RRU interface to the general purpose processor unit.

Power is expected to be supplied as defined Section 7.2.

Antenna interface.

Configuration programming (e.g., location installation)

## Separation/Isolation

Avionics equipment must be installed with separation requirements outlined in FAR 25.1309 and other appropriate standards.

Regarding aircraft with carbon fiber composite fuselage, the data obtained from the Airbus A350 shows there is no risk to the structure if the system produces local temperatures of no more than 95° C.

## Interoperability

The RRU is a component of the radio system. The radio system provides both TSO and aircraft-specific (non-TSO) functionality. The aircraft-specific functionality may include the maintenance system interface, data loading system interface, analog or digital audio system interface, data link system interface, etc. The same supplier/entity should supply both the Remote Radio Unit that hosts the TSO functionality and the aircraft-specific software that may be hosted in a general-purpose computing platform that may not necessarily be provided by the same supplier. The physical interface between the Remote Radio Unit and the general-purpose computing platform should be standardized.

# Antennas

## Installation

Currently, most CNS antennas are mounted on the fuselage surface. Some antennas (e.g., localizer and glideslope antennas) are located under the nose radome and others are located under the Vertical Tail Plane skin (e.g., HF Comm. and VOR). The antennas are mounted using threaded fasteners. Various methods are used in order to reduce moisture intrusion and for corrosion prevention, including the use of primer, sealant, or gaskets. Besides, their proper bonding is ensured by means of the gaskets or the fasteners.

Aircraft mounted antennas require a ground plane. A ground plane is simply the ‘grounded’ side of the antenna. A signal is radiated by the antenna by generating a difference in potential between the antenna and a ground reference. The ground reference does not have to be a highly conductive surface to serve as the ground plane. Note that for externally mounted antennas, some types of composite fuselages (e.g., carbon fiber) can still act as a ground plane even though they are much less conductive than an aluminum skin.

Antenna installations also need to be designed to minimize impacts due to lightning strikes during aircraft operations. Different means are provided in order to provide lightning protection. This includes ensuring that there is a conductive path running from the antenna baseplate to the mounting fasteners to the mounting nut plates to structure (or a Current Return Network on composite airplanes). It is also common practice to have a coaxial cable disconnect within a few feet of the antenna. This allows lightning currents that travel down the outer shield of the coaxial cable to be shed to ground at the disconnect, preventing these currents from traveling to the RF transmitter/receiver and causing damage.

Antennas which have a large baseplate area (e.g., TCAS antennas and Satcom antennas) are mounted using an antenna adapter plate in order to match the aircraft’s radius of curvature. The adapter plate will be flat on the antenna side that matches the antenna’s flat baseplate, and the side facing the fuselage will be curved in order to match the fuselage’s curvature. The adapter plate can be furnished by the airframer as part of the antenna installation or by the supplier of the antenna.

CNS system installations may be affected by Precipitation static (P-static). P-static is a term used to describe interfering noise resulting from the redistribution of charge on an operating aircraft. As an aircraft moves through the air, it acquires charge until sufficient voltage levels are reached to initiate a discharge. This discharge may occur between different parts of the aircraft or between the aircraft itself and the external environment. The noise or “static” resulting from these discharges can cause interference to sensitive aviation equipment resulting in the possible loss of certain communication or navigation capabilities for several minutes. The mechanisms that may create P-static interference are corona discharge from aircraft extremities, surface streamering or pin-hole discharge on non-conductive surfaces, and sparking between un-bonded conductive surfaces or components. All of these mechanisms have the potential to radiate energy into the frequency bands where Automatic Direction Finder (ADF), HF Communications, VOR, and VHF Communications operate. The energy from P-static discharge quickly diminishes with increasing frequency above VHF Communications (i.e., above 137 MHz). Experience has shown that P-static interference does not occur within higher frequency aircraft receivers. Due to the sensitivity of the VHF communication receivers, non-interference demonstrated by VHF Communication receivers is commonly used to indicate non-interference for the less sensitive VOR and ILS localizer receivers (in the 108 to 118 MHz band).

Aircraft design and installation considerations must be made in order to reduce P-static interference. Corona discharge from the aircraft can be controlled by static dischargers positioned on the aircraft extremities where the electric fields are the highest. Surface streamering on non-conductive surfaces exposed to airflow can be controlled by use of a conductive coating (paint or mesh) applied over the exposed surfaces. Lastly, sparking between conductive exposed surfaces and components can be controlled by proper electrical bonding and grounding.

## Power

Most CNS antennas are passive and do not require input power. However, some current CNS antennas do have internal amplifiers that receive power. These include:

- ARINC 743 Active GNSS Antennas which receive power via the coaxial cable center conductor.

ARINC 781 Inmarsat Satcom HELGA which receives power via the coaxial cable center conductor and Inmarsat Satcom HGA which receives power via a dedicated input from aircraft power.

ARINC 771 Iridium Satcom ALGA and HGA which receive power via the coaxial cable center conductor.

## Environmental Requirements

Table 8-1 – Antenna Environmental Requirements

| Environmental Requirements | ED-14/ DO-160 Section | Category | Applicability Criteria | Airframe Requirements |
| --- | --- | --- | --- | --- |
| Temperature & Steady State Altitude | 4 | D2 | Unpressured and uncontrolled temperature areas operating up to 50,000 ft. (15,200 m) MSL |  |
| Temperature Variation | 5 | A | External/unpressurized area of the aircraft; 10° C/min (minimum) |  |
| Humidity | 6 | C | External humidity environment. |  |
| Operational Shock | 7 | A / D / B / E | All zones | (1) |
| Crash Safety Impulse | 7 | B, E | If antenna detaches from installation in the event of a crash, it could cause injury to people or damage to equipment belonging to fuel, hydraulic, or oxygen systems, whose leakage could fuel a fire during a crash event | (1) |
| Operational Vibration | 8 | R | For equipment for which resistance to the effects of long exposure to vibration is required | (1) |
|  |  | H | High-level, short duration transient vibration encountered during engine fan blade loss. | (1) |
| Explosion Proofness | 9 | N/A | Not required for any CNS system |  |
| Waterproofness | 10 | R | Antenna subjected to driving rain or where water may be sprayed on it from any angle. |  |
|  |  | S | Antenna subjected to heavy stream of fluid such as de-icing, washing or cleaning operations |  |
|  |  | W | Antennas installed in internal areas (e.g., nose radome) where condensation occurs |  |
| Fluids Susceptibility | 11 | F | Antennas installed in all areas | (2) |
| Sand & Dust | 12 | S | Antennas installed in all areas |  |
| Fungus Resistance | 13 | F | Antennas installed in all areas |  |
| Salt Fog | 14 | T | Antennas exposed directly to the air during normal aircraft operations |  |
| Magnetic Effects | 15 | Z, A, B, or C | Category is dependent on equipment location and distance from magnetic compass: Z: D ≤ 0.3m A: 0.3m < D ≤ 1m B: 1m < D ≤ 3m C: D ≥ 3 m Antennas located close to the flight deck that could affect the standby magnetic compass. |  |
| Power Input | 16 | A(CF), A(NF), A(WF) and A | For active antennas that receive power directly from aircraft power. Primary power is from a constant or variable frequency ac system and where the dc system is supplied from transformer-rectifier units. | (3) |
| Voltage Spike | 17 | A | For active antennas that receive power directly from aircraft power. High degree of protection against damage by voltage spikes. | (3) |
| Audio Frequency Conducted Susceptibility – Power Inputs | 18 | R(CF), R(NF), R(WF) and R | For active antennas that receive power directly from aircraft power. Primary power is from a constant or variable frequency ac system and where the dc system is supplied from transformer-rectifier units. | (3) |
| Induced Signal Susceptibility | 19 | C() or Z() depending upon the installation and the aircraft’s primary power source | C() is for installations with long wire runs or minimum separation | (4) |
| Radio Frequency Susceptibility | 20 | W / R | Test Cat. W required for EMH Category A. Test Cat. R required for EMH Category B, C, D | (4) |
| Radio Frequency Emissions | 21 | L, M,H, P or Q | Category is dependent on equipment location. | (4) |
| Lightning Indirect Effects | 22 | Per DO-160G §22.3 |  | (4) |
| Lightning Direct Effects | 23 | 1A/1B/1C/2A/2B/3N | Test category depends on the location of the antenna on the fuselage. Test applies to externally mounted antennas (not under the radome). In addition, antenna should not break apart such that it would create a hazard to the rest of the aircraft. |  |
| Icing | 24 | A |  |  |
| Electrostatic Discharge | 25 | A |  |  |
| Flammability | 26 | N/A |  |  |
| Smoke & Toxicity | -- | N/A |  |  |
| Hail | -- | N/A | Antenna 100% functional after the test | (5) |

Notes:

- For equipment + fixture; each airframe manufacturer has specific vibration and acceleration requirements (including PRA: Nose Wheel Imbalance, Fan Blade Out, and Sustained Engine Imbalance).

Each airframe manufacturer has specific list of fluids and temperatures.

Some Satcom configurations with HGA require power supply from the aircraft for the antenna. Each airframe manufacturer has a specific list of requirements for the antenna power when it is supplied directly from aircraft through a dedicated cable.

Each airframe manufacturer has specific requirements. Test levels vary depending on safety objectives and criticality (i.e., DAL), as well as Electromagnetic Hazard (EMH) category of the equipment.

Each airframe manufacturer has specific requirements besides the ED-14 / DO-160.

## Connectors

Circular connectors, such as the MIL-DTL-38999 series, are expected to meet the functional requirements and the environmental conditions for antennas that require coax connections (for RF interfaces) as well as data and power connections. Insert arrangements for MIL-DTL-38999 series connectors are in accordance with MIL-STD-1560. If no data or power interfaces are required, then TNC or N type connectors would be sufficient to support coaxial cable connections.

At present, collaboration between the CNS antenna supplier and the airframe manufacturer is necessary to select a connector that meets specific system requirements. Due to harsh environments in the crown area (top of the aircraft) and the bilge area (bottom of the aircraft), it is important for the connectors to be qualified to operate in these areas. The crown area is susceptible to “rain-in-the-plane” whereby condensation develops between the cold outer skin of the aircraft and the insulated cabin. The bilge area is prone to water and contaminants collecting as these flow down the sidewalls of the airframe. Antenna connectors in these areas have been prone to moisture intrusion. Threaded TNC or N connectors and MIL-DTL-38999 connectors with improved sealing should be used in these areas. In addition, taller connectors have previously been used in order to provide improved accessibility.

## Interfaces

The following interfaces are expected:

- RF interface to the Remote Radio Unit (RRU)

- Power input from aircraft power for those antennas that require it (e.g., Inmarsat Satcom HGA)

## Separation/Isolation

Avionics equipment and associated wiring must be installed to meet all applicable regulatory requirements including FAA 14CFR 25.1309, EASA CS 25.1309, and airframe manufacturer’s requirements. A system safety assessment is performed in order to ensure independence between functions, systems, or equipment which may be required in order to satisfy these requirements. Individual failure modes and external events are evaluated to ensure compliance. This includes identifying wire separation and system redundancy.

RF isolation recommendations for CNS systems are contained in a number of existing ARINC characteristics. These recommendations are provided in order to ensure that multiple transceivers and receivers can be used simultaneously on the same aircraft. Note also that some systems use a suppression bus in order to ensure non-interference and protection of RF circuitry.

## Interoperability

The CNS system should provide interoperability with associated ground or satellite systems, as applicable. It should provide interoperability with legacy CNS systems including existing infrastructure. In addition, each of the CNS systems should be interoperable with the other CNS systems. In some instances, filtering may be needed to ensure interoperability. For example, GNSS systems require filtering to prevent Inmarsat SATCOM induced intermodulation interference.

## Interchangeability

It is an objective that each antenna be able to operate with RRUs manufactured by different suppliers. Currently, a number of different CNS radios (e.g., HF Comm, VHF Comm, ILS Localizer and Glideslope, GNSS, VOR, DME, ATC Transponder, etc.) operate with a common antenna for that system. There are some systems (e.g., TCAS and Inmarsat Satcom) whereby the supplier of the radio is also the supplier of the antenna. The goal should be that the antenna support radios from multiple suppliers.

- SUMMARY OF CONCLUSIONS and recommendations

  - Introduction

This section provides conclusions reached based on the information documented within this report. It also provides recommendations for further work scope (if any is required).

  - Conclusions

The CNS Distributed Radio architecture concept was evaluated to determine the feasibility of distributing the RF and system processing sections of CNS radio systems. Based on the data and information gathered during the development of this report, the following conclusions, which identify the advantages and disadvantages of a Distributed Radio architecture, can be reached:

Advantages of a Distributed Radio Architecture

- Locating the RF transmit/receive (TX/RX) and related signal processing portions of the CNS radio in an RRU as close as possible to the antenna:

  - Reduces RF cable loss, and thus, minimizes RF design complexity

  - Negates the need for long coaxial cable runs from the antenna to the EE bay. This provides the benefits of reduced installation costs and weight.

- Hosting the remaining digital portion of the system on a general purpose computing platform, such as Integrated Modular Avionics (IMA), maximizes utilization of shared resources and reduces the need for Electronics Bay space. This provides the benefits of reduced cost and SWaP.

Challenges of a Distributed Radio Architecture

- Predicted reliability of a Remote Radio Unit (RRU) may  be less than current Electronics Bay installed CNS LRUs. This is primarily due to:

  - Increase in temperatures within the cabin crown area

  - Lack of forced-air cooling

  - Restriction in the use of cooling holes due to condensing moisture

  - Conductive thermal dissipation means sufficient to dissipate the expected thermal energy of CNS radios is deemed insufficient

- Installation of RRUs in remote areas (such as the crown or lower lobes) significantly increases the complexity of maintenance. Access doors would need to be installed in order to allow maintenance or removal & replacement of RRUs. It would be difficult to access the crown area with passengers onboard and difficult to access lower lobe areas with cargo loaded. With the predicted reliability being lower for RRUs (see above), this makes the RRU installation even less desirable.

Based on the above list of advantages and disadvantages of a CNS Distributed Radio architecture, a general conclusion can be made that the Distributed Radio architecture is not yet suitable (at this time) for commercial transport aircraft. The predicted, reduced RRU reliability along with challenging maintenance accessibility, as compared to existing EE bay mounted CNS LRUs, makes the Distributed Radio architecture a less desirable option.

A step-wise transition is the preferred migration path to CNS Distributed Radio implementation.

  - Recommendations

There are fundamental environmental and maintenance access issues that would need to be overcome in order to make this architecture feasible. If RRUs and their installation could be made more robust, and reliability could be significantly improved, then this could address one of the more significant issues. However, maintenance accessibility for an increasing number of RRUs could be a more challenging problem. Wholesale implementation may not be feasible at the time of this writing.

An alternative to the Distributed Radio concept would be the continued, increased integration of CNS radios. This integration takes advantage of using common RF TX/RX and processing (in the EE bay) and can include common antennas as well. For example, there is already an ARINC 768 Integrated Surveillance System (ISS) whereby the ATC Transponder and TCAS II radio functions have been integrated. The AEEC is currently working on an ARINC 768A architecture which further integrates the Distance Measuring Equipment (DME) system into the ISS architecture. This removes the need for both the separate RF TX/RX and processing in a separate ARINC 709 DME interrogator as well as the need for dedicated DME antennas. Another example is the ARINC 755 Multi-Mode Receiver (MMR) which has integrated the ILS, GNSS, GLS, VOR, and Marker Beacon functions by utilizing common RF RX and processing (in the EE bay). Airlines preference on CNS radio integration need to be considered in new aircraft planning.

Other opportunities for integrated RF and processing should be evaluated. This is especially true for not yet implemented systems such as the L-band Digital Aeronautical Communication System (LDACS).

Glossary Definitions

Acronyms and Abbreviations

ACAS-X	Airborne Collision Avoidance System

ACMS	Aircraft Condition and Monitoring Systems

ADS-B	Automatic Dependent Surveillance – Broadcast

AeroMACS	Aeronautical Mobile Airport Communication System

AIRB/CDTI	Airborne Situational Awareness/Cockpit Display of Traffic Information

AMC	Acceptable Means of Compliance

ATC	Air Traffic Control

BITE	Built In Test Equipment

CAVS	CDTI Assisted Visual Separation

CDN	Common Data Network

CDS	Cockpit Display System

CDTI	Cockpit Display of Traffic Information

CNS	Communication, Navigation, and Surveillance

DAL	Design Assurance Level

DME	Distance Measuring Equipment

DSP	Digital Signal Processing

EB	Expanded Beam

EE	Electrical and Electronics

EMH	Electromagnetic Hazard

EMI	Electromagnetic Interference

FHA	Functional Hazard Assessment

FIM	Flight-deck Interval Management

FMS	Flight Management System

GBAS	Ground Based Augmentation System

GLS	GNSS Landing System

GNSS	Global Navigation Satellite System

GPP	General Purpose Processor

GPS	Global Positioning System

HF	High Frequency

IFF	Interrogate Friend or Foe

ILS	Instrument Landing System

IM	Inner Marker

IMA	Integrated Modular Avionics

ITP	In-Trail Procedure

LDACS	L-band Digital Aeronautical Communication System

LNA	Low Noise Amplifier

LRRA	Low Range Radio Altimeter

LRM	Line Replaceable Module

LRU	Line Replaceable Unit

MB	Marker Beacon

MEL	Minimum Equipment List

MM	Middle Marker

MOPS	Minimum Operational Performance Standard

MSL	Mean Sea Level

MTBF	Mean-Time Between Failure

MTBUR	Mean-Time Between Unscheduled Removal

OM	Outer Marker

OMD	Onboard Maintenance Documentation

OMS	On-Board Maintenance Systems

PFD	Primary Flight Displays

PTP	Precision Time Protocol

RA	Resolution Advisory

RF	Radio Frequency

RFI	Radio Frequency Interference

RRU	Remote Radio Unit

RTP	Real-Time Protocol

Satcom	Satellite Communication

SSR	Secondary Surveillance Radar

SURF	Surface Situational Awareness

SWaP	Size, Weight, and Power

TA	Traffic Advisory

TACAN	Tactical Air Navigation

TAWS	Terrain Awareness and Warning System

TCAS	Traffic Collision Avoidance System

TSN	Time Sensitive Network

TSO	Technical Standard Order

VDB	VHF Data Broadcast

VHF	Very High Frequency

VOR	VHF Omni-directional Range